| Hardware Circuit Testing Sample | Software Testing Sample | Communication Testing Sample | Machine Learning Testing Sample | Mobile App Feature Testing Sample |
| --- | --- | --- | --- | --- |
| Test Name:; PCB Continuity Testing. Performed By:; Mohammed Ghazal. Time/Date; 15th of March – 4:00pm. Test Description:; In this test, we use the multimeter’s continuity test to ensure current flow in PCB connections without shorts or open circuits. Steps:; - Place the design of our PCB next to the fabricated one. - Build a table of all connections starting and ending points. - Turn the knob on the multimeter to select the continuity test. - Connect the leads between the starting and ending points in each connection. - Listen for the continuous sound.. Expected Results:; All PCB connections are correct with no short or open circuits. Observed Results:; Connection AF is loose. Consider re-soldering or new PCB.. Acceptance Criteria:; Fail. Test Result:; Connection; Starting Pt; Ending Pt; Result. PD0 to In1; PD0; In1; Pass. PD1 to In2; PD1; In2; Pass. B7; B; 7; Pass. AF; A; F; Fail. …. ….. | Test Name:; Testing the digital filter implementation code on Arduino. Performed By:; Mohammed Ghazal. Time/Date; 15th of March – 4:00pm. Test Description:; The purpose of the test is to validate the digital filter designed to process the signal from the sensor. In the test, the output from the sensor x[n] is displayed on the screen. Also, the output of the filter is displayed, i.e., y[n]. If the code works, y[n] should be showing less variations than x[n] because h[n] is a low-pass filter. The code is doing convolution between x[n] and h[n] to produce y[n] using a for loop and if-statements to avoid errors and crashes.. Steps:; - Populate h[n] with the filter coefficients. - Populate x[n] with dummy data. - Produce y[n] by convolving x[n] and h[n]. - Print y[n] values. - Perform the convolution between h[n] and x[n] on Matlab to produce y[n] - Compare y[n] from Matlab from the conv() command with the y[n] calculated using our Arduino code.. Expected Results:; y[n] calculated using our code should match the y[n] produced by Matlab with. Observed Results:; Values calculated from the Arduino implementation are sometimes 5% different from the ones in Matlab.. Acceptance Criteria:; less than 1% error. Test Result:; Failed. Must debug and check the code and the math.. | Test Name:; Testing the communication between the Arduino node and the mobile application. Performed By:; Mohammed Ghazal. Time/Date; 15th of March – 4:00pm. Test Description:; The purpose of the test is verifying that the communication protocol messages are delivered to and from the Arduino and the mobile application.. Steps:; - Verify that on trigger the signal leaves the Arduino by monitoring the serial communication pins using the oscilloscope. - Test the Wifi connection between the Arduino and the router. - Send a message from the Arduino to the server and test if the database receives the message. - Test that the message is routed and displayed on the mobile application. - Verify that on trigger a message leaves the mobile app. - Ensure the message content reaches the server and is stored in the database. - Check if the message from the server reaches the Arduino. Expected Results:; Two-way communication between the Arduino and the mobile is tested. Latency is calculated and within acceptance criteria.. Observed Results:; - Message from Arduino is received on the mobile app within in 0.53s. - Message from the mobile is received on the Arduino in 2.3s. Acceptance Criteria:; - Message from Arduino is received on the mobile app in less than 1s of latency. - Message from the mobile is received on the Arduino in less than 1s of latency.. Test Result:; Failed due to latency from mobile to Arduino. | Test Name:; Testing the machine learning model performance. Performed By:; Mohammed Ghazal. Time/Date; 15th of March – 4:00pm. Test Description:; In this test, we apply K-fold cross validation to evaluate the machine learning model on a limited data sample.. Steps:; - Shuffle the dataset randomly. - Split the dataset into k groups of approximately equal size. - Take one group as a test data set. - Take the remaining groups as a training data set. - Fit a model on the training set and evaluate it on the test set. - Retain the evaluation score and discard the model - Summarize the performance of the model using the sample of model evaluation scores.. Expected Results:; The average of our k recorded accuracies is called the average testing accuracy which should be the performance metric of our model.. Observed Results:; Our model doesn’t perform equally on all folds of test sets (average testing accuracy is low), then we’re likely overfitting.. Acceptance Criteria:; - The difference between one-fold accuracy and another shouldn’t exceed 10%. - Accuracy > 90% - Precision > 90% - Recall > 90%. Test Result:; Our model doesn’t generalize well from our training data to unseen data. We need to collect more balanced data or use regularization.. | Test Name:; Testing the functionality of the mobile application. Performed By:; Mohammed Ghazal. Time/Date; 15th of March – 4:00pm. Test Description:; The purpose of this test is to test the geolocation recording feature of the mobile app.. Steps:; - Press on the record geolocation data button on the UI interface - Move outdoors for 1km - Observe pins added on the map - Observe database to see locations are logged correctly. Expected Results:; All locations are recorded leading to successful tracking. Observed Results:; Locations are recorded at a frequency of 3 recordings per second.. Acceptance Criteria:; Locations are recorded at a frequency of 5 recording per second. All locations recorded on the phone make it to the database.. Test Result:; Pass. Works as expected.. |