AVIATION INSTITUTE OF MAINTENANCE

# PHYSICS

PROJECT G30

Name: ____________________						Date: ______________

## ITEM:	Use and understand principles of simple machines; sound, fluid, and heat dynamics; basic aerodynamics, aircraft structures; and theory of flight

STUDENT PERFORMANCE GOALS:

OBJECTIVE:	Use and understand the relationship between work, force, and power.

PROCEDURE:	1. Solve / answer the following problems.

2.  Answers must indicate in. lbs., ft. lbs, as applicable.

- Refer to figure A.  What is the amount of force applied to rope “A” to lift the weight shown? (You must show your calculations)

  - 12 pounds

  - 15 pounds

  - 20 pounds

  - 30 pounds

Calculation

Mechanical advantage= number of ropes sections supporting the load= 4

MA= Fo/Fi

MA=4

Fo= 60

Fi=?

Fi= Fo/Ma

= 60/4

=15

Answer= b. 15 pounds

- What force must be applied to roll a 120 pound barrel up an inclined plane 9 feet long to a height of 3 feet (disregard friction)? (You must show all of your work).

#### L/I=R/E

L= length of ramp, measured along the slope

I= height of the ramp

R= weight of the object to be raised or lowered

E= force required to raise or lower the object

- 40 pounds

- 120 pounds

- 360 pounds

- 393 pounds

Solution

E= RI / L

= 120 x 3 /9

= 40

Answer= a. 40 pounds

- Mechanical advantage may be calculated by dividing the weight by the __effort________.

MA =R / E

W / f = L / h

## Solve for the unknown variable in the following problems

4. Weight = 400 pounds, Effort =100 pounds		MA = __4________________

5. Effort = 500 pounds, Weight = 250 pounds		MA = __0.5________________

6. Mechanical advantage of 10, Weight = 1000 pounds, Effort = _100_pounds________________

7. Mechanical advantage of 2, Effort of 50 pounds, Weight = __100_pounds__________________

8. Determine the amount of force required at B in order to balance this lever.

A = 20 pounds

- Moment of A = _____300lb.in___________

- Moment of B = ___30Blb.in_____________

- B =                   ___10 _____________ pounds

Use above figure for question #8 & #9

9. If B = 50 pounds, what would box A have to weigh for this lever to be in balance?

- Moment of B = ___1500______________

- Moment of A = ___15 A______________

- Weight of A =   ____100 pounds_____________

10. Find the force required to roll a 500-pound airplane up a 20-foot ramp that is 5 feet high.

- W = _______________

- f =  ____125lbs____________

- L = ________________

- h = ________________

(Figure is used for questions #11-#15)

11. What is the mechanical advantage of this pulley system?  ____2____________

12. How much force would be required to lift a 1000-pound weight? ____500 pounds_____________

13. How much weight could be lifted if an effort of 100 pounds were applied? ______200 pounds____

14. If the weight being raised was 2,000 pounds, and the rope was pulled 10 feet, how high

would the weight be raised? _____5 feet______________

15. To raise this 2,000 pound weight to a height of 2 feet, how much rope must be pulled?

_______4 feet_______________

16. If the fluid pressure is 2000 PSI in a 1 inch line supplying an actuating cylinder with

a piston area of 6 square inches, the force exerted on the piston will be:

a. 1,200 Pounds

b. 12,000 Pounds

c. 333.33 Pounds

d. 1.200 Pounds

Answer= b. 12,000 pounds

17. If the fluid pressure is 800 PSI in a ½ inch line supplying an actuating cylinder with

a piston area of 10 square inches, the force exerted on the piston will be?

a. 4000 pounds

b. 1600 pounds

c. 8000 pounds

d. 800 pounds

Answer= c. 8000 pounds

18. 88 degrees Fahrenheit is equal to:

a. 31.11 degrees Celsius

b. 3.111 degrees Celsius

c. 33.00 degrees Celsius

Answer= a, 31.11 degrees Celsius

19. 85 degrees Celsius is equal to:

a. 18.50 degrees Fahrenheit

b. 185.0 degrees Fahrenheit

c. 1.850 degrees Fahrenheit

d. 1,850 degrees Fahrenheit

Answer= b. 185 degree Fahrenheit

20. Provided with the following information about an aircraft in flight:

Altitude = 0 Feet

Temperature = 59 degrees Fahrenheit / 15 degrees Celsius

Pressure IN. HG. = 29.92

Aircraft is traveling @ 661.2 Knots

At approximately what speed in MPH is the aircraft flying?

a. 770.61 MPH

b. 745.70 MPH

c. 761.70 MPH

d. 770.45 MPH

Answer= c. 761.70 MPH