Analysis of motion of a paradrop

Question 1

Motion of a paradrop can be categorized into two stages

The first stage (stage1) is where the parachute ha not opened up. Here motion is a free fall and the only factor affecting motion is gravity.

The equation of motion and free body diagram are given as follows:

s=12gt2+vot+s0

v(t)=gt+vo

Where g is the gravitational acceleration g=9.81m/s^2

S is the altitude from which the paradrop is dropped

V is the velocity and t is time in seconds

Free body diagram

Stage2

In this stage the parachute opens up and the influence of air resistance in terms of drag force comes in .

The drag force is given by the following equation:

Fdrag=12ACd⍴v2

Where, A is the area of the parachute in m, Cd is the drag coefficient while ⍴ is the density of air in kg/m3.

The equation of motion is given as follows:

ma=mg-Fdrag

ma=mg-12ACd⍴v2

Dividing with m on both sides:

a=g-12ACd⍴v2m

But

a=ⅆvⅆt

And let     k= 12ACd⍴ assuming the density of air is constant and the parachute is fully opened.

ⅆvⅆt=g-kv2m

The above equation can be expressed in this form:

ⅆvⅆt+kv2m=g

Which is a first order ODE.

The solution of the above equation is given as follows:

vt=mgk(e-kmt-1)

s=12at2+vot+s0

Therefore:

s(t)=-12g(e-kmt)t2+mgke-kmt-1t+s0

With the above equations we are able to get the altitude and the landing velocity when solved.

Free body diagram

Question 2

The landing time and velocity when the parachute is not activated.

We use equation for stage 1 and solve them to get time and landing velocity if we know the altitude.

s=12gt2+vot+s0

v(t)=gt+vo

Using the first equation we can solve for the landing time

vo=0

s0=the altitude at which the parachute is launched

g=9.81m/s^2

Therefore

t=22sg

And

v(t)=gt+0

Question 3

The equations in the finite difference form can be written as shown below:

ⅆvⅆt+kv2m=g

vi+1-viΔt+kmvi=g

The initial conditions is that v (0) =0; s (0) = s0

Question 4

The matlab code for solving the finite difference equation is as follows:

N=10;

g=9.81;

m=80;

k=14.7875;

a=1;

b=m/k;

c=-g*m/k;

t=linspace(0,1,N+1);

t1=linspace(0,1,N-1);

dt=t(2)-t(1);

A=eye(N-1);

A=A*(k/m-1/dt)

A=A+1/dt*diag(ones(N-2,1),1)

v=((A)^-1)*(M)

plot(t1,v)

xlabel('time')

ylabel('velocity')

plot of velocity against time is as follows:

Velocity increases up to maximum value which is the terminal velocity.

Before the parachute is activated the plot of height against time is as shown below

The height decreases due to the influence of gravity

The matlab code is as follows

s0=3000;

g=9.81;

v0=0

t=linspace(1,10,10000)

s=-0.5*g.*t.^2+v0.*t+s0;

plot(t,s)

xlabel('time')

ylabel('height')

After the parachute is activated velocity is as follows

Velocity decreases to zero.

The matlab code for the above graph is as follows:

m=80;

g=9.81;

k=14.7875;

t=linspace(1,20,10000)

v=-sqrt(m*g/k)*tanh(sqrt(k*g*t/m))

plot(t,v)

xlabel('time')

ylabel('velocity')

OVERAL VELOCITY IS AS FOLLOWS