Assignment Part II Wk1

Student’s Name

Institutional Affiliation

Assignment Part II Wk1

- Describe when oxygen (O2) therapy is needed

Oxygen therapy is needed based on three basic ways:

Use of laboratory measures such as aerial blood gas testing to document hypoxemia

Specific clinical problem or condition

Tachypnea, tachycardia, cyanosis or distressed overall appearance resulting from hypoxemia. (Page 907 under Assessing the Need for Oxygen Therapy)

- Assess the need for O2 therapy

Oxygen therapy is needed when PaO2 is less than 35 to 60 mm Hg or Oxygen saturation (SaCO2) is less than 87% to 90% to subjects breathing room air. (Page 907 under Assessing the Need for Oxygen Therapy)

.

- Describe what precautions and complications are associated with O2 therapy

Oxygen Toxicity, currently referred to as hyperoxic acute lung injury. This condition is characterized by as a PaO2 less than 300 mm Hg.

Depression of Ventilation. When breathing moderate to high O2 concentrations, a very small percentage of patients with chronic obstructive pulmonary disease (COPD) and chronic hypercapnia may hypoventilate.

Retinopathy of Prematurity. Also called retrolental fibroplasia, is an abnormal eye condition that occurs in some premature or low-birth weight infants who receive supplemental O2

Absorption Atelectasis. FiO2 values greater than 0.50 present a significant risk of absorption atelectasis.

Fire Hazard. Fires involving enriched 02 environs continue to occur in health facilities, despite numerous preventive measures. Oxygen is highly flammable and any leakages makes the orating rooms prone to fire. (Pages 908 and 909)

- Select an O2 delivery system appropriate for the respiratory care plan

I would select the high flow system since it always exceeds the patient’s flow and provides a fixed FiO2. (Page 910 under Delivery Systems: Design and performance)

- Describe how to administer O2 to adults, children, and infants

When it comes to adults, a Nasal cannular is to be used. The cannular consists of disposable plastic tips or prongs connected to several feet of small bore O2 supply tubing. Prongs are inserted into the vestibule of the nose while supply tubing is connected directly to a flow meter or bubble humidifier.

For children, a Nasal Catheter is the most appropriate. It is a soft tube with several small holes at the tip that is inserted by gently advancing it along the floor of either nasal passage and visualizing it just behind and above the uvula.

For infants, Transtracheal catheter is used. A Transtracheal catheter is a thin polytetrafluoroethylene catheter inserted into the trachea between the second and third tracheal rings, secured by a chain necklace. (Pages 911 - 914)

- Describe how to identify and correct malfunctions of O2 delivery systems

If there is no gas flow felt coming from the cannula, it can be corrected by adjusting the flow meter, checking connections, finding the obstruction or using an alternative device.

Mouth breathing is an indicator of habitual mouth breathing or blocked nasal passages. You should switch to a simple mask or venturi mask. (Page 915 table 42.5)

- Assess and monitor a patient's response to O2 therapy

The most frequent problems involve barotrauma to closed body cavities, such as the middle ear or sinuses. Pneumothorax and air embolism also are possible during oxygen therapy.

Early signs of impending CNS toxicity include twitching, seating, pallor, and restlessness and later seizures and convulsions. (Page 927 under indications)

- Describe how and when to modify or recommend modification of O2 therapy

The most common indications of O2 therapy are air embolism and carbon monoxide poisoning.

Typical therapy for air embolism involves immediate pressurization in air to 6 ATA for 15 to 30 minutes. This step is followed by decompression to 2.8 ATA with prolonged O2 treatment.

When it comes to carbon monoxide poisoning, the situation of the patient improves quickly with HBO treatment is HBO is the fastest way to remove carbon monoxide from the blood. (Page 928 under Box 42.8)

- Describe how to implement protocol-based O2 therapy

A pulse oximetry saturation of 92% is the point at which therapy is initiated. The patient is assessed each shift for the need of supplemental O2, which is adjusted depending on need. When the SpO2 is consistently 92% or greater on room air, therapy is discontinued. (Page 926 paragraph 3)

- Identify the indications, complications, and hazards of hyperbaric O2 therapy

(Pages 927 – 930)

Indications of hyperbaric O2 therapy include:

- Decompression sickness

- Air or gas embolism

- Carbon monoxide and cyanide poisoning

- Acute traumatic ischemia

- Acute peripheral arterial insufficiency

Complications of O2 therapy

- Air embolism. It is a condition that can occur with certain cardio vascular procedures, lung biopsy, hemodialysis, and central line placement. HBO can decrease the size of air bubbles which may reach the cerebral or cardiac circulation and can cause death.

- Carbon monoxide poisoning. CO poisoning accounts for approximately half of all poisoning deaths in the United States. The condition of a patient with CO poisoning improves quickly with HBO treatment since HBO is the fastest treatment to remove CO from the blood

Complications of O2 therapy include;

- Central nervous system toxic reaction

- Pulmonary toxic reaction

- Sudden decompression

- Reversible visual changes

- Claustrophobia

- Decreased cardiac output

One of the common hazards is fire.

- Identify when and how to administer specialty therapeutic gases

NO is administered when there is need to improve oxygenation and reduce the need for extracorporeal membrane oxygenation.

NO is administered to mechanically ventilated patients through a system with the capability for operator-determined concentration of NO in the breathing gas a constant concentration throughout the breathing cycle, and a concentration that does not cause generation of excessive iNO2.