| ITEM No. | DESCRIPTION BILL OF QUANTITIES FOR A COMFERENCE HALL | UNIT | QUANTITY | RATES($) | AMOUNT($) |
| --- | --- | --- | --- | --- | --- |
| A B C D E F G H I J ITEM A B C D E F G I J ITEM A B C D E F ITEM A B C D E F ITEM A B C D E F A B | SUBSTRUCTURES Excavations and Earthworks Excavate oversite average 2500 mm deep to remove vegetable soil, load, wheel and deposit 100 meters away and later spread and level on site where directed. Excavate to reduce levels starting from stripped level and column pits not exceeding 2.5M deep. Trim and prepare bottoms of excavations to receive blinding. Remove surplus excavated material and cart away. Return fill and well ram selected excavated material around foundations Fillings Approved murram filling to make up levels, well rammed and consolidated in 150mm thick layers. 300mm thick approved hardcore filling spread, levelled, well rammed and consolidated in 150mm thick layers to receive concrete surface bed Level and blind surface of hardcore with 50mm quarry dust to receive concrete surface bed. Anti - termite treatment as "Premise 200 sc" distributed by High Chemicals essentials or other equal and approved applied to surface of blinded hardcore and surrounding areas strictly in accordance with manufacturer's instructions (subject to a TEN YEAR GUARANTEE to the satisfaction of the Architect.) Mass Concrete class 15/20 J 50mm Thick blinding: under foundations DESCRIPTION SUBSTRUCTURES CONTINUED Vibrated reinforced concrete class 100/100mm in; Strip foundation 300mm Thick ground floor slab Sawn formwork to: Sides of strip foundations Edge of floor bed 75-150mm high Reinforcement Fabric Mesh reinforcement Fabric mesh reinforcement to B.S. 4483 ref: A93 weighing 2.22 Kg per square meter in floor slab (measured net - no allowance for minimum 230mm laps) including tying wire and supports as required Foundation walling Rough dressed natural stones or other equal and approved bedded and jointed in cement sand mortar (1:4) 250 mm thick walls Hessian based bituminous felt or any other equal and approved dump proof course:- Horizontal: 200mm wide. 1000mm gauge polythene dump proof membrane with 300mm welted lap under concrete. Plinths 150mm thick mortar cement and sand (1.4) rendered to plinths Prepare and apply three coats of bituminous: Ditto TOTAL FOR SUBSTRUCTURE CARRIED TO SUMMARY DESCRIPTION SUPERSTRUCTURE WALLING Masonry walls bedded and jointed in cement and sand 1:4 in: 300mm thick wall reinforced with 32x2 mm hoop iron every alternative course Provide 100mm pipe vents in beam above all doors with wire gauze and 25mmx400mm approximately in all windows upper middle compartment as permanent ventilation Ring beam Concrete class 20/20 for tong beam Reinforcement High yield reinforcements bars Y16 Y10 Binding wire Formwork Sawn formwork for lintel DOORS Double door with transom size 1500x2100mm high (D1 External door) 1.6mm thick middle rails complete with handles bracings to and all necessary accessories(ironmongery) including priming with one coat of red oxide primer before delivery to site and all weld grounds. Timber door inclusive of architraves, quadrants, locks, hinges and door frames WINDOWS Supply and fix steel casement windows consisting of standard Angline section frames and square rodes primed with one coat red oxide complete DESCRIPTION 10mby 2.5m high (W1) 5m by 1.5m high(W2) 0.8by 0.8m high (W3) Inclusive of brushes Glazing 5mm thick clear sheet glass and glazing to metal in metal patty in panes 0-10 – 0-50 square meters Ditto obscure glass ditto Window board Wrot cypress 125x25mm window board 20mm quadrant Curtain rods 25mm diameter chrome rods 20mm diameter chrome net rod Painting Touch up manufactures priming coat prepare and apply two undercoats and one finishing coat gloss oil paint to:- Glazed metal surface (measured Ove glass) TOTAL COLLECTION FOR SUPERSTRUCTURES CARRIED TO SUMMARY ROOF CONSTRUCTION Slab roofing inclusive of all metal rods and painting FINISHES Internal wall finishes 40mm thick 1:4 cement sand render to wall DESCRIPTION 30mm thick cement screed to wall Wall tiles 250 by 200mm wall tiles Cement Grote white Painting Prepare and apply three coat first quality plastic emulsion paint to:- Internal walls External wall finishes 40mm thick 1:4 cement sand render to ring beam Painting Prepare and apply three coat first quality plastic emulsion paint to; Ring beam TOTAL FOR WALL FINISHES CARRIED TO SUMMARY FLOOR FINISHES 40mm thick 1:4 cement and sand render to floor 5mm thick cement screed to floor Floor tiles 300mm square floor tiles Cement tiles Grote white TOTAL FOR FLOOR FINISHES CARRIED TO SUMMARY Electrical Installation Works Allow a prime cost sum of Kenya shillings one hundred thousand for electrical works Mechanical Works Allow a prime cost sum of Kenya shillings one hundred and thirty thousand for mechanical works. | SM CM SM CM CM CM SM SM SM SM UNIT CM SM SM SM SM SM LM SM SM SM UNIT SM NO CM Kg Kg roll SM NO NO UNIT NO NO NO SM SM LM LM LM LM SM SM UNIT SM ctns bags SM SM SM SM SM ctns bags | 240 288 192 198 96 120 72 200 200 116 QUANTITY 24 200 76 24 200 248 192 200 56 56 QUANTITY 272 48 20 500 300 8 228 15 25 QUANTITY 8 5 6 36 2.4 28 28 28 28 40 660 QUANTITY 660 120 20 20 660 272 120 200 200 200 100 40 | 50 100 100 4 2 9 18 5 3 5 RATES($) 11 13 35 18 24 13 1 2 4 30 RATES($) 15 2 10 15 15 15 40 115 80 RATES($) 65 50 20 14 20 15 15 3 2 4 4 RATES($) 10 5 6 4 2 4 10 4 2 7 6 2 | 12000 28800 19200 792 192 1080 1296 1000 600 580 AMOUNT($) 264 2600 2660 432 4800 3224 192 400 224 1680 82016 AMOUNT($) 4080 96 200 7500 4500 120 9120 1725 2000 AMOUNT($) 520 250 120 504 48 420 420 84 56 160 31923 15000 2640 AMOUNT($) 6600 600 120 80 1320 1088 1200 13648 800 400 1400 600 80 3280 20000 15000 |

| ITEM | DESCRIPTION THREE BEDROOM UNIT SUMMARY |  | AMOUNT (Ksh) |
| --- | --- | --- | --- |
| 1 2 3 4 5 6 7 8 TOTAL | PRELIMINARIES SUBSTRUCTURES SUPERSTRUCTURE ROOF CONSTRUCTION WALL FINISHES FLOOR FINISHES ELECTRICAL WORKS MECHANICAL WORKS | FROM PAGE 1 “ “ 2 “ “ 4 “ “ 4 “ “ 5 “ “ 5 “ “ 6 “ “ 6 | #REF 82016 31923 20000 13468 3280 20000 15000 185683$ |

BILL OF QUANTITIES

CONFERENCE HALL

OCTOMBER 2017