CIT 360 Spring 2020

Project #2

Due by Friday March 6th 2020 (Week x) during lecture (see below)

This project should be performed in groups of 2 students. Please be sure to prepare one single submission with both team members’ names on it.

Background: Creating a Bootable USB and CD/DVD for Clonezilla Backup/Restore

In this exercise you will install Windows 7/8 OS onto virtual computer, then use the Clonezilla utility to make a backup image of the installed OS and save the backup image onto a separate virtual disk.  Later, you will restore the backup image to the original disk and boot from the restored image to confirm the correctness of the backup and restore operations.

Clonezilla is a disk copy/clone utility from the Taiwanese National Center for High Performance Computing (NCHC), Steven Shiau, author.  It is available as a free download from http://clonezilla.org.  Clonezilla works with disk blocks, which means that it backs up both files and metadata that define partitions and volumes.  It supports imaging and copying of a very long list of different file systems.  As a point of comparison, Clonezilla is similar to an older commercial utility called Norton Ghost.  Ghost was acquired by Symantec and has now been replaced by Symantec System Recovery (SSR).

There are several modes of operation, but the one we will use in the lab is Clonezilla Live.  This is a version that is installed onto a small bootable Debian Linux installation on either an optical (CD/DVD) or USB flash drive. In other words…

Clonezilla + Debian Live = Clonezilla Live

As part of getting some experience working with the utility, you should create both a CD and a USB version of Clonezilla Live.  The Clonezilla website contains detailed instructions which will be summarized below, under Prep.

General “Your Computer” Lab Policies

Do whatever you want with your own stuff.

Prep

- Mount the class file server:  \\casper.ecs.csun.edu\cit360

  - If you connect from outside campus, you will need to first create a VPN connection to CSUN.  The Cisco VPN client software is available for free download from the CSUN ITR software downloads page at

    - https://www.csun.edu/itr/downloads/index.cgi (requires CSUN ID logon)

  - If you connect to a Windows file share from Mac OS, use

    - smb://casper.ecs.csun.edu/cit360

- Logon with the following credentials:

  - User ID:  casper.csun.edu\download

  - Password:  d0wn!oad

- Download a copy of the Windows 7 and/or Windows 8 installation disks.

  - Disks will be a little over 3 GB each.

  - HINT: For this lab, you may find it best to use the 32-bit (x86) ISOs.

  - You may perform the lab with EITHER Windows7 or Windows8, or both if you want practice. You must select AT LEAST ONE OF THESE OSes.

- Go to the clonezilla.org website.

- Follow the instructions to create and burn a Clonezilla Live disk.

  - Basically, you will download the image from the website and burn it to a CD-R (about 131 MB).

- Follow the instructions to create a Clonezilla Live bootable USB.

  - Download the tuxboot.exe utility to the local machine.  This utility can be run in place without having to install it.

  - You can either point Tuxboot to the Clonezilla ISO that you have already downloaded, or you can point it to the latest distribution on the Clonezilla website.

  - Plug in your Clonezilla USB and indicate it to Tuxboot.

  - Tuxboot will now install Clonezilla Live to the USB.

Lab Work

- Install Windows 7/8 onto the first hard drive.

- Logon to Windows as admin.

- Use the disk administration utility to confirm that the second disk is

  - NTFS

  - Empty

- Create a folder at the top level on the second disk to hold Clonezilla backup images, with an obvious name.

  - e.g., “TeamVenture_Diaz_Salazar_Clonezilla_Images”

- Shutdown

- Boot from Clonezilla CD or USB (ultimately confirm that you can boot to Clonezilla from both).

- Make a backup of the Windows 7/8 on the first disk and store the image on the second disk.

- Reboot Windows 7/8.

- Modify the OS so that it is not identical to the backup.

- Shutdown.

- Boot from Clonezilla.

- Restore the Windows 7/8 image from the 2nd disk to the 1st disk (overwriting the existing OS on the first disk).

- Shutdown.

- Boot from Windows 7/8 on the first disk.

- Confirm that the OS is the restored version not the modified version.

Clonezilla Guidelines for Creating a Backup Image

Clonezilla is a complex program with a lot of runtime options.  Here is a guide to get you started on the backup operation.

- Pick “English” or whatever. If you pick something else, I likely won’t be able to help you.

- Don’t touch the keymap.

- Start Clonezilla.

- Choose the “Device  Image” option.

- Mount the Clonezilla image directory.  This must be the 2nd hard drive, the empty drive with the Clonezilla images folder you created earlier.

- Choose “local-dev” since the 2nd disk is a local device, not a network device.

- If you were using a USB as your backup image device, there are special instructions here, but it doesn’t apply for this exercise.

- You will get a list of devices, for example:

sda1	953 MB ext4

sda3	140 GB ext4

sdb1	149 GB ntfs

You must carefully choose the correct device at this point.

- On this device, locate the directory you created earlier.  If you can’t find it, this is a good opportunity to bail out, you’ve probably chosen the wrong disk.

- You will get a quick confirmation of disk space available for that image directory.

- Choose “beginner’s mode” which comes with lots of default values for options.

- Select the “savedisk  save disk to an image” mode

- Give the image a name.

- Choose local disk as source (the disk to be backed up, which is different from the disk holding the backup images)

- Choose “skip checking/repairing source file.”  (Or not, and wait through it).

- Choose to skip or check after image is saved.

- At this point, the full command-line version of the command is echoed. (You can copy it and enter it next time directly from the command prompt, saving you the trouble of going through the command builder phase.)

- Clonezilla will collect some info now.

- Clonezilla is ready to make the backup.  It will prompt you several times to make sure you picked the right devices.

- Commence backup.

For the restore operation, you are on your own.

Credit for Project

You will show your machine to the instructor, who will confirm that the proper steps were performed.  Be ready to answer a few simple questions about what you did.  Credit for this project is basically a check-off that your team performed the work.