Virginia States University

Department of Engineering and Computer Science

ENGR 102-01 – Introduction to Engineering (Fall 2020)

Date:

Chapter 7 HW: Fluid Mechanics

On my honor, I have neither given nor received any unauthorized aid on this homework, and I pledge that I am in compliance with the VSU Honor System.

Print Name	                                                                      Signature

Instructions: Show your work to get full credit; partial credit will be given where appropriate. Start your solution by identifying the appropriate equation (s).

- The specific gravity of a liquid is 0.920. Find the density and specific weight of the liquid in SI and English units. (10 points)

Let  x = unknown density

y = unknown specific weight

Sg = specific gravity = 0.920

Density of Water ρ(water) = 1000 kg/m³(SI Unit)  = 1.940 slugs/ft³ (English Unit)

SI Units

Density of Unknown Liquid

x = Sg * ρ(water) = 0.920(1000 kg/m³) = 920 kg/m³

English Units.

x = Sg * ρ(water) = 0.920(1.940 slugs/ft³) = 1.78 slugs/ft³

Specific Weight of Water

λ(water) = 9.810 kN/m³ (SI Unit) = 62.4 lb/ft³ (English Unit)

Specific Weight of Unknown Liquid

SI Units

y = Sg * λ(water) = 0.920(9.810 kN/m³) = 9.02 kN/m³

English Units

y = Sg * λ(water) = 0.920(62.4 lb/ft³) = 9.02 kN/m³ = 57.408 kN/m³

- A 12-cm diameter cylindrical can is filled to a depth of 10 cm with motor oil (ρ=878 kg/m3). Find the mass and weight of the motor oil. (10 points)

r = radius = 6cm

h = height = 10cm

V = Volume = πr2h= π*62*10=1130.97 cm3

M = p* v =  0.878 g/cm3* 1130.97 cm3=992.9946 g=0.993 kg

W = mass * gravity = 0.993 kg * 9.81 = 9.7413 N

Mass = 0.993kg

Weight = 9.7413 N

- Find the  volume of mercury (sg = 13.55) that weighs the same as 0.04 m3 of ethyl alcohol (sg = 0.802) (5 points)

- Find the pressure change required to produce a 1.90 percent decrease in the volume of benzene at 20°C. (5 points)

Bulk modulus of water (k)  is 2×109N/m2

∆Pk=∆VV

∆P=1.9%*k  = 0.019 * 2×109N/m2  = 3.8 ×107 N/m2

∆P=3.8 ×107 N/m2

- What is the percent change in the volume of a liquid whose bulk modulus value is 100 times the change in pressure? (5 points)

∆P=x

Bulk module = 100 x

∆Pk=∆VV

x100x=∆VV

Percent change in the volume = x100x*100%=1%

- A storage tank containing heavy fuel oil (ρ=906 kg/m3) is filled to a depth of 5 m. Find the gauge pressure at the bottom of the tank. (5 points)

Pressure = density * height* gravity

ΔP = γ h = ρ g h

= 906 kg/m3*5 m * 9.81 m/sec2 = 44,439.3 Pa

= 44,439.3 Pa

- The deepest known point in the oceans of the earth is the Mariana Trench, east of the Philippines, with a depth of approximately 10.9 km. Taking the specific gravity of seawater as sg = 1.030, what is the pressure at the bottom of the Mariana Trench? Express your answer in kPa, psi and atmospheres. (10 points)

Patm = 1.013 * 105Pa

P = 1.10 * 108 	kPa

1 Pascal = 0.00014503773 Psi.

1.10 * 108 Pa = 15954.151 Psi

1atm = 1.01325 * 105Pa

1.10 * 108 Pa  =  1.10 * 108Pa *1atm1.01325 * 105Pa  = 1085.6 atm

P = 1.10 * 108 	kPa

= 15954.151 Psi

= 1085.6 atm

- The side of a barge is submerged 6 m below the surface of the ocean (γ=10.1 kN/m3). The length of the barge is 90 m. Treating the side of the barge as a plane vertical surface, what is the total force exerted by the ocean on the side of the barge? (10 points)

FR= γ h A/2

h = 6m

L = 90m

W = 6m

FR= 10.1 kN/m3* 6m *90m* 6m* 0.5 = 16,362 kN

Total force exerted by the ocean on the side of the barge =  16,362 kN

- A glass tube carries mercury at an average velocity of 30 cm/s. If the inside diameter of the tube is 4.0 mm, find the volume flow rate and the mass flow rate. (10 points)

Volume flow rate = Area * Average Velocity

Mass flow rate = mass / unit time

4 mm = 0.4cm

Area = π*0.424 = 0.1256 cm2

Volume flow rate = 0.1256 cm2* 30 cm/s = 3.77 cm3/s

Density for mercury = 13.6 g/cm3

Mass flow rate  = Volume flow rate * Density for mercury

= 3.77 cm3/s * 13.6 g/cm3= 51.272 g/cm3

Volume flow rate =3.77 cm3/s

Mass flow rate  = 51.272 g/cm3

- A furnace requires 1250 lbm /h of cold air for efficient combustion. If the air has a specific weight of 0.064 lbf/ft3, find the required volume flow rate. (10 points)

Volume flow rate = Area * Average Velocity

Mass flow rate -= 1250 lbm /h

1 lbm /h = 0.000125998 kg/s

1250 lbm /h = 0.1575 kg/s

specific weight = 0.064 lbf/ft3

1 lbf/ft3 = 16.0185 Kg/ m3

0.064 lbf/ft3= 1.025 kg/ m3

Volume flow rate = Mass flow rateDensity

0.1575 kg/s 1.025 kg/m3= 0.1575 m31.025s = 0.153 m3/s

Volume flow rate = 0.153 m3/s

- A nozzle is a device that accelerates the flow of a fluid. A circular  nozzle that converges from an inside diameter of 5 cm to 3.5 cm carries a gas at a volume flow rate of 0.10 m3/s. find the change in average velocity of the gas. (10 points)

5cm = 0.05 m

35cm = 0.35m

Volume flow rate = Area * Average Velocity

Average velocity in 5 cm pipe

0.10 m3/s =  π*0.0524 * V1

V1 = 50.93 m/s

Average velocity in 3.5 cm  Pipe

0.10 m3/s =  π*0.03524 * V2

V2 = 103.94 m/s

Change in average velocity = V2- V1

= 103.94 – 50.93 = 53.01m/s

- A pipe branch is illustrated in schematic form below. Find the mass flow rate m4. Does the fluid in branch 4 enter or leave the junction.

Incoming fluid = outgoing fluid

M1 + M3 + M4 = M2

10 +6 +m4 = 18

M4 = 18 – 16 = 2

M4 = 2Kg/s

Positive sign of M4 shows that the fluid in the branch 4 will be entering the junction.

Useful Equations and Equivalence Statements

- ρ=mV

- W=mg

- γ=WV

- γ=ρg

- sg=ρfluidρH2O,   4℃                	  ρH2O,   4℃=1000 kg/m3

- sg=γfluidγH2O,   4℃                	  γH2O,   4℃=9810 N/m3

- K=-∆P∆V/V

- ΔP = γ h

- ΔP = γ h = ρ g h

- FR= γ h A/2

- zcp = h/3

- =vA

- m=ρV

- V = v A = v π D2/4

- m=sgHgρH2OV

- v = V/A =  VWH

- v1/v2 = (V/A1)/(V/A2) = A2/A1 = (D2/D1)2