Virginia States University

Department of Engineering and Computer Science

ENGR 102-01 – Introduction to Engineering (Fall 2020)

Date:

Chapter 7 HW: Fluid Mechanics

On my honor, I have neither given nor received any unauthorized aid on this homework, and I pledge that I am in compliance with the VSU Honor System.

Print Name	                                                                      Signature

Instructions: Show your work to get full credit; partial credit will be given where appropriate. Start your solution by identifying the appropriate equation (s).

- The specific gravity of a liquid is 0.920. Find the density and specific weight of the liquid in SI and English units. (10 points)

- A 12-cm diameter cylindrical can is filled to a depth of 10 cm with motor oil (ρ=878 kg/m3). Find the mass and weight of the motor oil. (10 points)

- Find the  volume of mercury (sg = 13.55) that weighs the same as 0.04 m3 of ethyl alcohol (sg = 0.802) (5 points)

- Find the pressure change required to produce a 1.90 percent decrease in the volume of benzene at 20°C. (5 points)

- What is the percent change in the volume of a liquid whose bulk modulus value is 100 times the change in pressure? (5 points)

- A storage tank containing heavy fuel oil (ρ=906 kg/m3) is filled to a depth of 5 m. Find the gauge pressure at the bottom of the tank. (5 points)

- The deepest known point in the oceans of the earth is the Mariana Trench, east of the Philippines, with a depth of approximately 10.9 km. Taking the specific gravity of seawater as sg = 1.030, what is the pressure at the bottom of the Mariana Trench? Express your answer in kPa, psi and atmospheres. (10 points)

- The side of a barge is submerged 6 m below the surface of the ocean (γ=10.1 kN/m3). The length of the barge is 90 m. Treating the side of the barge as a plane vertical surface, what is the total force exerted by the ocean on the side of the barge? (10 points)

- A glass tube carries mercury at an average velocity of 30 cm/s. If the inside diameter of the tube is 4.0 mm, find the volume flow rate and the mass flow rate. (10 points)

- A furnace requires 1250 lbm /h of cold air for efficient combustion. If the air has a specific weight of 0.064 lbf/ft3, find the required volume flow rate. (10 points)

- A nozzle is a device that accelerates the flow of a fluid. A circular  nozzle that converges from an inside diameter of 5 cm to 3.5 cm carries a gas at a volume flow rate of 0.10 m3/s. find the change in average velocity of the gas. (10 points)

- A pipe branch is illustrated in schematic form below. Find the mass flow rate m4. Does the fluid in branch 4 enter or leave the junction.

Useful Equations and Equivalence Statements

- ρ=mV

- W=mg

- γ=WV

- γ=ρg

- sg=ρfluidρH2O,   4℃                	  ρH2O,   4℃=1000 kg/m3

- sg=γfluidγH2O,   4℃                	  γH2O,   4℃=9810 N/m3

- K=-∆P∆V/V

- ΔP = γ h

- ΔP = γ h = ρ g h

- FR= γ h A/2

- zcp = h/3

- =vA

- m=ρV

- V = v A = v π D2/4

- m=sgHgρH2OV

- v = V/A =  VWH

- v1/v2 = (V/A1)/(V/A2) = A2/A1 = (D2/D1)2