# Circuits

Name: ____________________________

Open the simulation:

https://phet.colorado.edu/sims/html/circuit-construction-kit-dc/latest/circuit-construction-kit-dc_en.html

Double click on the “Lab” box.

Familiarize yourself with the simulation. Play around with the electric components. Drag the wire, battery, light bulb from the panel on the left side and try to build a circuit. When you are finished testing all the settings, click on the orange reset button on the lower right corner.

Step 1: Now build the circuit shown below using a light bulb, a battery and some wires.

Click on the battery in your circuit, and you will see a battery scale appear. Adjust the battery voltage to 6 volts.

Step 2: Drag the ammeter from the panel on the right-hand side and place its crosshair on the wire next to the light bulb to measure the current through the light bulb. The ammeter will display the current. Write down this value below.

Current through the light bulb: I =

Step 3: Use the voltmeter to measure the voltage across the light bulb by touching the

red lead of the voltmeter on one side of the light bulb where the light bulb is connected to the positive terminal of the batter and the black lead to the other side of the light bulb that is connected to the negative terminal of the battery. Write down this voltage below.

Voltage across the light bulb:		V =

Step 4: According to the Ohm’s law: 		I= VR

Use the Ohm’s law along with measured values of V and I to calculate the resistance of the light bulb (assuming that the wires are of negligible resistance). Show all your work below.

Resistance of the light bulb:	R (calculated) =

Now click on the light bulb in your circuit, the light bulb resistance will be displayed. Write it down below.

R (actual) =

Does your calculated value match with the actual value?

Your answer:

Step 5: Click on the “schematics components” button link  located on the lower right corner (above the reset button) and notice how all the components in your circuit are replaced by the standard symbols used in the textbooks. We will use the simulation in the “schematic components” view mode for the rest of this lab.

Step 6: Build the circuit shown below using three light bulbs, battery, and some wires.

Step 7: Click on the battery and set the battery voltage to 12 volts. Click on the light bulb on the left and set its resistance to 10Ω. Similarly set the resistance of the light bulb in the center equal to 20Ω and that of the right-hand side one equal to 30Ω.

Step 8: Use the ammeter to measure the current through each of the light bulbs (by placing the cross hair of the ammeter on wire either before or after the light bulb. Write down the values of these currents below.

I1 (through the light bulb on the left) =

I2 (through the light bulb in the center) =

I3 (through the light bulb on the right) =

Now measure the total current flowing from the battery by placing the cross hair of the ammeter next to the battery.

Itotal (from the battery) =

Step 9: Use the voltmeter to measure the across each of the light bulbs. Write down the values of these voltages below.

V1 (across the light bulb on the left) =

V2 (across the light bulb in the center) =

V3 (across the light bulb on the right) =

Step 10: These light bulbs are connected in series.

Do you notice any pattern in the currents through the light bulbs connected in series? Explain.

Your answer:

Do you notice any pattern in the voltages across the light bulbs connected in series?

Your answer:

Is there any relationship between the voltages across each light bulb and its resistance? Explain.

Your answer:

Is there any relationship between the battery voltage and the voltages across the light bulbs? Explain.

Your answer:

Step 11: Now build the circuit shown below using three light bulbs, battery, and some wires.

Step 12: Click on the battery and set the battery voltage to 12 volts. Click on the light bulb on the top and set its resistance to 10Ω. Similarly set the resistance of the light bulb in the center equal to 20Ω and that of the one at the bottom equal to 30Ω.

Step 13: Use the ammeter to measure the current through each of the light bulbs (by placing the cross hair of the ammeter on wire either before or after the light bulb. Write down the values of these currents below.

I1 (through the light bulb on the top) =

I2 (through the light bulb in the center) =

I3 (through the light bulb at the bottom) =

Now measure the total current flowing from the battery by placing the cross hair of the ammeter next to the battery.

Itotal (from the battery) =

Step 14: Use the voltmeter to measure the across each of the light bulbs. Write down the values of these voltages below.

V1 (across the light bulb on the top) =

V2 (across the light bulb in the center) =

V3 (across the light bulb at the bottom) =

Step 15: These light bulbs are connected in parallel.

Do you notice any pattern in the voltages across the light bulbs connected in parallel?

Your answer:

Do you notice any pattern in the currents through the light bulbs connected in parallel? Explain.

Your answer:

Is there any relationship between the current through each light bulb and its resistance? Explain.

Your answer:

Is there any relationship between the total current flowing from the battery and the currents through the light bulbs? Explain.

Your answer:

Step 16: Now build the circuit shown below using two light bulbs, battery, a switch, and some wires.

Adjust the battery voltage to 12 volts, resistance of the top light bulb to 12Ω and the bottom light bulb to 6Ω.

Keeping the switch open (bottom light bulb disconnected), use the ammeter to measure the current through each light bulb and the total current flowing from the battery.

I1 (through the light bulb on the top) =

I2 (through the light bulb at the bottom) =

Itotal (from the battery) =

Step 17: Now close the switch and measure the current through each light bulb and the total current flowing from the battery again.

I1 (through the light bulb on the top) =

I2 (through the light bulb at the bottom) =

Itotal (from the battery) =

Notice that there is more current through the smaller resistance as you would expect from the Ohm’s law:

I= VR

But how would you explain the fact that by closing the switch even though you added another resistance in the circuit, the total current flowing from the battery increases instead of decreasing?

Your answer:

Step 18: Ohm’s law is valid for individual resistors, but it is also valid for the entire circuit so that the total current flowing through the battery is given by:

Itotal= VtotalRtotal

Where Rtotal is the total or net resistance of the circuit. Rtotal for any number of resistors connected in parallel is given by:

1Rtotal=1R1+1R2+1R3+ .  .             (resistors connected in parallel)

We have only two resistors in our circuit (two light bulbs) so Rtotal will be given by:

1Rtotal=1R1+1R2

In our circuit: R1 = 12Ω and R2 = 6Ω. Use these values in the above equation and calculate Rtotal. Show all your work below.

Rtotal =

Now use the Ohm’s law to calculate the total current from the battery (remember Vtotal = 12 volts). Show al your work:

Itotal= VtotalRtotal

Itotal =

Does your answer match with the total current flowing from the battery that you measured in Step 17?

Your answer:

Step 19: Now, what would be the total or net resistance of the circuit if any number of resistors were connected in series? The answer is:

Rtotal=R1+R2+R3+ .  .             (resistors connected in series)

Let’s test this. Remember that in Step 6 we build the series circuit below:

Where R1 = 10Ω, R2 = 20Ω, and R3 = 30Ω. Find Rtotal using the formula above for the resistors connected in series. Show all your work below.

Rtotal =

Now use Ohm’s law to find the total current flowing from the battery. Show all your work below.

Itotal =

Does your answer match with the total current flowing from the battery that you measured in Step 7?

Your answer:

Step 20: Calculate the total current flowing from the battery in the circuit below. Resistors symbols are used instead of the light bulbs here.

Show all your work below:

Itotal =