Codes and Standards Assignment

Student’s Name

Indiana State University

Construction Project Management

CNST 450

21st Sept 2020

Question 1

The maximum staircase riser should be 7 inches, and the riser height should be vertically measured between the adjacent treads of the leading edges per R1009.3

Question 2

The minimum fixture clearance for a water closet should be 60 inches when perpendicular measured from the sidewall and 56 inches perpendicular measured from the rear wall as per R604.3.1.

Question 3

The minimum corridor width should be 44 inches as per 1018.2

Question 4

The minimum footing width for a two-story conventionally framed home on soil that is 2000 PSF should be 15 inches per R403.1.1

Question 5

As per R802.7.1, the notch should be a maximum of a sixth of the 2×10 dimensional lumber floor joist and should not be longer than a third of the depth of the 2×10 dimensional lumber floor joist. Additionally, it should not be placed in the middle one-third of the span. Therefore, the maximum notch should be 2 inches to the bottom or top of the lumber floor joist. The hole should also not exceed 2 inches to the notch.

Question 6

As per table R602.3.1, the maximum height for a 2×6 wall stud spaced 16 inches should be 8 feet for exterior and load-bearing walls or 10 feet for interior walls and non-loading walls.

In cases of snow loads being equal or less than 25 pounds per square foot and the design wind speed equal to or not more than 130mph, studs supporting a roof load with less than 6 feet tributary length should have a maximum height of 18 feet when spaced 16 inches at the center or 20 feet when the spacing at the center is 12 inches.