Video: How to Use the Dissection Scope

*You should read over the “Labeled Dissection Scope” sheet before viewing this video. This will familiarize you with the dissection scope parts and their functions.

# This document accompanies the online video titled Stereoscopic Binocular Microscope Tutorial - Boston University Biology. Watch and answer the questions as you proceed through the video.

# Note: the dissection scope is sometimes called a stereo microscope, or sometimes even a stereoscopic microscope. This type of scope allows a person to magnify an object that is already visible. However, greater detail will become evident.

# To use a dissection scope:

# 1) Plug the scope in to an outlet.

# 2) Use of any other material or substance may damage the lenses. The fibers present in other materials have been removed in a lens paper and therefore it is more suitable to clean lenses. Clean the eyepieces with lens paper. (Why should only lens paper be used to clean the lens?)

# 3) Turn on the scope. There are usually two lamps on a dissection scope: a top lamp, which will shine a light down on to the specimen and a bottom lamp, which can shine a light up through a thin specimen.

# 4) Place the specimen on the viewing plate (sometimes called the stage). The letter e is used as a specimen in this video.

# The knob on the right (called the zoom magnification knob) tells the magnification of the objective lenses, which are located inside the body of the scope. The numbers on this knob range from 0.7 through 4.5. The eyepieces (ocular lenses) have a magnification power of ten.

# Imagine the zoom magnification knob is set to 0.7. Take this number and multiply it by ten (the magnifying power of the eyepieces).  0.7 X 10 = 7. The object being viewed will be magnified seven times.

# How to Know the Total Magnification of an object on the Dissection Scope:

# Do this: Multiply the number setting on the zoom magnification knob by ten.

# Example: The zoom mag. knob is set to 2.0. Multiply this by 10 (eyepiece mag.)					     2.0 x 10 = 20 times magnified

# Adjust the eyepieces (oculars) so that you can look through the scope using both eyes. (See the explanation at 1:58 in the video.)

# Notice how you can adjust each eyepiece for to conform to your eyes.

# Notice the lichen that is viewed.

# Notice the difference in viewing at 15X magnification and 45X magnification.

# When finished using the dissection scope:

# Turn off the scope

# Remove the specimen

# Turn off and unplug the scope

# Wipe off the viewing plate

# Wrap up the cord, cover the scope and store the scope in the proper place.

# Questions:

# 1. __________________________Bottom lamp. Suppose you have half of a walnut and you want to look at the details. Which lamp would you use to illuminate the walnut, the top lamp or the bottom lamp?

# 2. The stage viewing plate is colorless the slide does not have any stain and can be more clear than the other side of the plate. The stage viewing plate is usually white side up. However, the stage plate can be turned over. What color is the other side of the stage plate? Why would you want to use that side?

# 3. __________36__You have set the zoom magnification knob to 3.6. What is the total magnification of your specimen?

# 4. Which of the following items would best be viewed using a dissection scope?

# _____	A slide of human red blood cells

# _____ An ant

# _____ A twig from a tree

# _____ A slide of bacteria that live in the human intestines

# _____ A colorful rock you found in the Huron River

# 5. Turn the focus knob up until the image is observed. Zoom out to reset it back to lower setting. Imagine you have just increased the magnification on your scope. What two adjustments should you make after you have done this? (If you’re not sure, check the video again at the 2:35 mark.)