# College of Engineering and Technology

# Mechanical Engineering

Project Deliverable 1

GRASSHOPPER BEAM ENGINE

Academic Year: 2020 – 2021

Semester: Spring 2021

Course Code and Title: ME 352 machine design I

| Prepared by: |  |
| --- | --- |
| Supervised by: |  |
| Dr. |  |

# Abstract

Since long time, the steam beam engine was first used at the time of industrialization. Thomas Newcomen, a blacksmith, managed in the early of the 1700s to develop the first truly promising steam engine to drive pumps to lift the water from mines in Cornwall. The efficiency of these engines was improved by several engineers like James Watt, Jonathan Hornblower, Arthur Woolf and William Murdoch. One of the different types of this engine is Grasshopper Beam Engine was developed by William Murdoch. In this paper, A brief history of this engine will be discussed, its design, its main component, material selection and the working theory. In this report, we will explain the optimal design of this machine, which leads to converting the alternating linear motion into a rotational movement, and the function of each piece of the device will be clarified and the materials that have been chosen in the manufacture of this piece as well as the method of manufacturing it.

Key words: Beam engine mechanism, Kinematic analysis, Rotational Motion, Simple Harmonic Motion, Piston, Cylinder.

Table of Contents

Abstract	1

List of figures	3

List of tables	3

Introduction	4

Design Concept Generation and Components’ Design	6

The base	6

The crankshaft	6

The beam	7

Crosshead	7

Moving leg	8

3.6. Cylinder and piston	8

Flywheel	9

Material selection	11

Stainless steel	11

Manufacturing procedure	13

Casting	13

Milling Machine	13

Drilling process	14

Kinematic analysis	15

Displacement analysis	15

Velocity analysis	18

Kinematic analysis	21

Kinetic analysis	21

References	24

# List of figures

Figure 1 Steam beam engine version	4

Figure 2 William Murdochs' steam Engine model	5

Figure 3 the Marine engine.	5

Figure 4 crankshaft	6

Figure 5 beam part	7

Figure 6 moving leg part	8

Figure 7 piston and cylinder	9

Figure 8 flywheel	10

Figure 9 Casting Process	12

Figure 10 milling operation	13

Figure 11 drilling machine	13

# List of tables

Table 1properties of mild steel	11

Table 2 properties of stainless steel	11

# Introduction

There are many types of steam engines, including the radiating engine which used to drive water pumps where the thermal energy in the steam is converted to a kinetic energy. This thermal energy is developed to apply coal fuel boilers in which steam is produced through which steam is then pumped. As the piston rises, the beam moves and the pump rod lower, cold water is injected into. the cylinder to produce condensed water, and finally, a vacuum is formed, which pushes the piston down and raises the pump rod again in each cycle. Figure 1 shows a model of the steam beam engine and its main parts. (Frenken, 2004)

Figure 1 Steam beam engine version

The earliest recorded example of a Grasshopper Beam Engine was William Murdochs' steam Engine model (figure 2.) from 1784. In this model the beam is pivoted at one end, rather than the center of mechanism. The beam provided negligible mechanical benefit and looks to be used in place of a crosshead because it was effectively a connecting rod return engine. The American engineer Oliver Evans drew a high-pressure marine engine (figure 3.) in 1801 and in 1805 built the Oruktor Amphibolos, an amphibious excavator. (Thurston, 1899)

Figure 2 William Murdochs' steam Engine model

Figure 3 the Marine engine.

The advantage of this engine is that the working end of the beam can be connected directly to the piston rod of the engine, and in this way the engine is made much more compact, especially in terms of height, since the space that the usual swing arm connections between the beam take up and the cross-beam of the Piston rod. The bar length can also be reduced. While the beam is generally at least three times the total stroke length in the normal steam beam engine that of the Grasshopper is generally about 1 3/4 times the length. The stroke length and its weight are also proportionally less. The beam is guided in a straight line by a parallel movement device which is suitably modified for this special purpose, for which it is necessary that the anchored end is capable of side movement. This was generally made in the form of a rectangular frame fused with a lattice or crossed struts. Despite the weight of the Grasshopper beam is completely unbalanced, it is much lighter than the ordinary beams enable engines of this type to operate fairly satisfactory at higher speeds than the previous type. The compactness of the entire engine structure provided inherent rigidity as well as simplified assembly and installation using cuboid or brick. However, the engines were often sunk or raised in the floor to make room for an air pump and condenser under the bed and a race was made available to make room for the steering wheel.

# Design Concept Generation and Components’ Design

## The base

The base or frame is a component that is attached to the floor. Holds the entire machine assembly on top.

## The crankshaft

Crankshaft is a mechanism of arms or levers to impart reciprocating motion to a rotating shaft. One end of the crank is attached to the shaft and the other end receives reciprocating motion from the hand, crank, etc.

The Shaft connects a lever to a crank or crankshaft in a steam beam engine. Together with the crank, it forms a simple mechanism that converts the reciprocating motion rotary motion into a rotary motion. A connecting rod can also convert a reciprocating motion into a rotary motion.

In figure 4. A clear understanding of how these working steps take place within the crankshaft

Figure 4 crankshaft

## The beam

The beam is the main part of the engine that connects all the parts of the engine together. It’s obvious why this type of engine is called beam engine. During the reciprocating movement of the piston cylinder, the beam transfers the movement to the rest parts of the engine. In this type of engine, the beam is pivoted at one end, rather than the center of mechanism which leads to a longer stroke of the piston and a greater output of force.

Figure 5 beam part

## Crosshead

It moves between guides between the middle of the beam and the engine body which prevents the piston rod from getting bent due to excessive mechanical loads.

## Moving leg

This part is connected between the beam and the engine base, it is connected to the beam at its fulcrum where it’s rotated.

Figure 6 moving leg part

## 3.6. Cylinder and piston

It is made of cast brass and is bolted to the frame at one end. In the case of small engines, it is integrated into the frame. It forms a chamber in which the piston moves back and forward due to the action of steam. One end of the cylinder is closed by the center of a separate cap and the end is known as the cylinder cap end. The other end, known as the crankshaft end, carries the stuffing box through which the piston rod passes. The piston is made of Stainless Steel. Force is transmitted due to the pressure of the steam that acts on the crosshead through the piston. The piston rod runs through the cylinder head. Prevents steam from escaping from the cylinder into the atmosphere.

Figure 7 piston and cylinder

## Flywheel

It has a unique advantage. It absorbs energy when supply exceeds demand, and releases energy when demand exceeds supply, thereby flattening power distribution and avoiding rapid fluctuations during rotation. Help the mechanism to overcome dead center positions.

Figure 8 flywheel

Figure 9 Design of mechanism

## Material selection

Mild steel is the least expensive of all steels and the most widely used steel. Mild steel is used in almost all types of steel products and it is weldable, very hard and, although it rusts easily, very durable. It contains less than 2 % carbon. Mild steel can be magnetized and used in most projects that require a large amount of metal. The material used for the base and crank shaft is steel is mild steel.

Table 1properties of mild steel

# Stainless steel

Stainless steel has many desirable properties because of its chromium content, it is extremely resistant to corrosion, high strength and durability, its high and low temperature resistance, increased formability and easy fabrication.

Table 2 properties of stainless steel

# Manufacturing procedure

## Casting

The casting process uses a wide range of metals, as it is an easy, simple and inexpensive method. Casting operations vary and there are some that are used for a one-time mold, and some of them are multi-use molds due to the quantity required for manufacturing. The casting process is based on converting the metal to a liquid state by raising its temperature to the melting temperature, and then this liquid becomes in molds that have been designed in a hollow form in the form of the piece to be manufactured. And then leave the mold with the molten metal in it until its temperature decreases and this molten metal turns into a solid piece of metal. Thus, the parts to be manufactured are obtained. This process is good for minerals that have a low melting point.

Figure 10 Casting Process

## Milling Machine

This process is used to shape metal parts. By means of the grinding machine in this machine, the metal piece is fixed. Then the cutter moves at different angles to form the piece on the desired shape. In this machine, the metal piece moves up and down, and the cutter moves left and right, back and forward. This process is also used to remove excess parts and result from the casting process.

Figure 11 milling operation

## Drilling process

This process is used to make holes in the metal pieces by means of the drill machine, and it is done by the drill that rotates and presses the metal pieces and that also presses this drill on the pieces and requires high pressure.

Figure 12 drilling machine

# Kinematic analysis

In this part we will show displacement analysis, velocity analysis, and acceleration analysis for this mechanism by graphical method.

## Displacement analysis

General position

First critical position

Second critical position

Displacement in links

| link | displacement |
| --- | --- |
| OA | 180° CW |
| OC | 12.8° CCW |
| OD | 0.5° CCW |
| DE | 15.73° CW |
| FF' | 90 cm |

## Velocity analysis

Assume ωOC=1rads

V=ωR=1*213=213cms

Scale 1:10

| link | R cm | V cm/s | 𝜔 rad/s |
| --- | --- | --- | --- |
| OC | 213 | 213 | 1 |
| DC | 174 | 212.77 | 1.222816 |
| OD | 161 | 28.89 | 0.179441 |
| CB | 63 | 77.03741 | 1.222816 |
| BA | 158 | 252.41 | 1.597532 |
| OA | 32 | 360.09 | 11.25281 |
| OE | slider | 43.413 | slider |

Acceleration

an=V2R

| link | R cm | V cm/s | 𝜔 rad/s | an | a |
| --- | --- | --- | --- | --- | --- |
| OC | 213 | 213 | 1 | 213 | 1029 |
| DC | 174 | 212.77 | 1.222816 | 260.1786 | 4015 |
| OD | 161 | 28.89 | 0.179441 | 5.18405 | 4116 |
| CB | 63 | 77.03741 | 1.222816 | 94.20259 | 5328 |
| BA | 158 | 252.41 | 1.597532 | 205.143 | 4478 |
| OA | 32 | 360.09 | 11.25281 | 405.654 | 8953 |
| OE | slider | 43.413 | slider | slider | 4961 |

# Kinematic analysis

# Kinetic analysis

Figure 15 static force analysis

F=150 KN

For link 2

F=0

F+F12+F32=0

Fj+F12i-F32cos12 i-F32sin12j=0

For i

F12-F32cos12 =0

For j

F=F32sin12

150=F32sin12

F32=721  KN

F12=705 KN

For link 3

F=0

F43=F23=-F32=-721  KN

F63=721  KN

F53=721  KN

For link 4

F=0

F14=F14=-F43=-721  KN

For link 5

F=0

F15=F35=-F53=-721  KN

For link 6

F=0

F16=F36=-F63=-721  KN

# Conclusion

We have done a study on this device and know its parts, how it works and the movement that each part performs in it, in order to convert the reciprocating movement resulting from the pressure of the steam into a rotational burning that is used. A study was conducted to find out the best materials that are used in the manufacture of those parts and the industrial processes that are used, and an analysis of displacement, velocity and acceleration was made.

# References

Frenken, K. (2004). The early development of the steam engine: an evolutionary interpretation using complexity theory. Industrial And Corporate Change, 13(2), 419-450. doi: 10.1093/icc/dth017

Thurston, R. (1899). THEORY OF THE STEAM ENGINE. Science, 9(227), 659-659. doi: 10.1126/science.9.227.659

A model single cylinder Grasshopper beam engine,. (2018). Retrieved 23 April 2021, from https://www.christies.com/en/lot/lot-769604

Polly Model Engineering: Stationary Engine Kits - Anthony Mount Models. (2021). Retrieved 23 April 2021, from https://www.pollymodelengineering.co.uk/sections/stationary-engines/anthony-mount-models/easton-Andersons-Grasshopper-Beam-Engine.asp