General observations

- Historical demand growth: associated with quality of life (consumer electronics)

- Thermodynamic efficiencies: Primary vs. secondary energy consumption

- From 1901 through 1970 robust increases in demand, doubles between 1960 and 1970

- The “New Deal” created the TVA, the Rural Electrification Administration and the Bonneville Power Administration

- The industry grew dramatically in the 1940s and '50s, thanks to high demand for power

- By the mid-'60s utilities were accustomed to steadily rising demand.

- Presumption of inelastic demand

- Long-term power demand has grown steadily since the 1980s, but investments in transmission have lagged

- Challenges to continued growth:

- Environmentalists demands

- Cost of generating electricity from coal also rose with pollution regulation

- Costs of nuclear power and TMI

- Cash-strapped utilities reduced their investments in transmission systems.

Basics in electricity system

- Stages

  - Production

  - Transmission

  - Distribution

- Production/generation

- Producers: IOUs, publicly owned utilities, federally owned (e.g. TVA. Bonneville Power Authority, IPP (includes cogeneration)

- 1 GWe enough power for a city of  1,000,000 people

- Coal 37%, natural gas 30%, nuclear 19%, hydro 8%, wind 3%, other 3%

- Economics

- Economies of scale

- Fixed costs – incurred w/out regard to how much power is produced

- Variable costs – fuel costs, fuel to energy conversion factors

- Ramping costs – start up time and shutting down

- Least cost dispatching, plants with the lowest variable costs over time of use.

- Reliability and load

- Base load

- Peak load/reserve margins/excess capacity

- Transmission

  - Capital costs and natural monopoly

  - Transmission losses  friction and heat, as high as 10%, heat capacity of lines

- Loop flow problem  flow across all available lines, demand and congestion, interconnected users

- Need for co-ordination  load management, load balancing, synchronized command and control operations

- Grids and control areas  North American Electric Reliability Council (NERC); RTOs (e.g. PJM)

- Developed in response to historic blackouts (1965 blackouts across eight Northeast states and Ontario province; 1977 25-hour blackout in New York City; 2003, the largest blackout in the nation's history plunged 50 million people in the Northeast, Midwest and Ontario, Canada caused an estimated $10 billion in business disruptions)

- Need for capital investment and expansion

- Distribution

  - From high voltage transmission lines to usable lower voltage electricity

  - Large scale initial investment, low marginal costs to add new customers

- Policy Agenda

- Regulation and Restructuring

- Regulatory federalism

- Demand management

- Grid Modernization