| Sheet name | Description |
| --- | --- |
| Wright's Model | Example of Wright's model that allows you to change the learning rate and cost of initial unit. |
| Average cost of X2 units | Calculation of cumulative average cost for X2 units when you have information on the average cost for the first X1 units and an assumed learning rate. |
| Incremental costs | Calculating the incremental cost of increasing cumulative production from X1 units to X2 units. |
| Estimating learning rate, 1 | Estimating the coefficient b and the learning rate from two data points. |
| Estimating learning rate, 2 | Estimating the coefficient b and the learning rate from more than two data points. |

| Wright's Learning Curve Model |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| In Wright's model, the doubling of production results in a decrease in cumulative average cost equal to 1 minus the learning rate. For example, with a learning rate of 80%, cumulative average costs with 2 cumulative units produced will be 80% of the cost of the first unit, and the cumulative average cost with 4 units produced will be 80% of the cumulative average with 2 units produced. |  |  |  |  |  |  |  |  |
| Learning rate (e.g, 0.80 for an 80% learning curve) |  |  |  | 0.80 |  |  |  |  |
| Resulting equation in the form Y = aX^b, |  |  |  | Y = aX^-0.322 |  |  |  |  |
| where Y = cumulative average time or cost per unit |  |  |  |  |  |  |  |  |
| X = cumulative number of units produced |  |  |  |  |  |  |  |  |
| a = time or cost to produce the first unit |  |  |  |  |  |  |  |  |
| b = slope of the function when plotted on log-log paper |  |  |  |  |  |  |  |  |
| = log of learning rate / log of 2 |  |  |  |  |  |  |  |  |
| = natural log of learning rate / natural log of 2 |  |  |  |  |  |  |  |  |
| You can plug numbers in below to see how it affects total cost, |  |  |  |  |  |  |  |  |
| Learning rate |  | 0.90 |  |  |  |  |  |  |
| Cost to produce the first unit |  | $100 |  |  |  |  |  |  |
| Value of b |  | -0.152 |  |  |  |  |  |  |
| Equation |  | Y = 100X^-0.152 |  |  |  |  |  |  |
| Cumulative Output, X |  | Cumulative Total Cost |  | Cumulative Average Cost |  | Marginal Cost |  |  |
| 1 |  | $100 |  | $100 |  | $100.00 |  |  |
| 2 |  | $180 |  | $90.00 |  | $80.00 |  |  |
| 4 |  | $324 |  | $81.00 |  | $68.69 |  |  |
| 8 |  | $583 |  | $72.90 |  | $61.82 |  |  |
| 16 |  | $1,050 |  | $65.61 |  | $55.64 |  |  |
| 32 |  | $1,890 |  | $59.05 |  | $50.07 |  |  |
| 64 |  | $3,401 |  | $53.14 |  | $45.07 |  |  |
| 128 |  | $6,122 |  | $47.83 |  | $40.56 |  |  |
| 256 |  | $11,020 |  | $43.05 |  | $36.50 |  |  |
| 512 |  | $19,836 |  | $38.74 |  | $32.85 |  |  |
| 1024 |  | $35,705 |  | $34.87 |  | $29.57 |  |  |
| Calculation of average cost of producing X2 units |  |  |  |  |  |  |  |
| This is useful if you want to know if producing and selling X2 units will be profitable. The average cost of producing X2 units can be compared to the price received. |  |  |  |  |  |  |  |
| Assumed learning rate (e.g, 0.80 for an 80% learning curve) |  |  | 0.80 |  | <=== input value |  |  |
| Value for b: |  |  | -0.322 |  | Calculated from input value |  |  |
| Average cost of producing the first X1 units: |  |  | $100 |  | <=== input value |  |  |
| Value for X1: |  |  | 10 |  | <=== input value |  |  |
| Value for X2: |  |  | 200 |  | <=== input value |  |  |
| Cumulative average cost for producing 200 units: |  |  | $38.12 |  | ===> desired answer |  |  |
| Variation: You can use the above to find what quantity is needed to reach a given cumulative average cost. |  |  |  |  |  |  |  |
| Target cumulative average cost: |  |  | $35.00 |  | <=== input value |  |  |
| From here, use Excel's goal seek function to set this cell to 0: |  |  | $3.12 |  | Goal seek this cell to 0. |  |  |
| by changing cell D13. |  |  |  |  |  |  |  |
| Required cumulative production: |  |  |  |  | ==> resulting answer when D22 = 0. |  |  |