HW 6

Chapter 6

- If the switch S7 of the input PORT0 connected to the data line D7 is at logic 1 and the other switches are at logic 0, specify the contents of the accumulator when the instruction IN PORT0 is executed.

MVI A,9FH

IN PORT0

MOV B,A

OUT PORT1

HLT

- Specify the output at PORT1 and the contents of the register B after executing the instructions in Problem #1.

- Specify the register contents and the flag status as the following instructions are executed.

| A | C | S | Z | CY |
| --- | --- | --- | --- | --- |
| XX | XX | 0 | 0 | 0 |

MVI A,5EH

ADI A2H

MOV C,A

HLT

- Specify the register contents and the flag status as the following instructions are executed.

| A | B | S | Z | CY |
| --- | --- | --- | --- | --- |
| XX | XX | X | X | X |

SUB A

MOV B,A

DCR B

INR B

SUI 01H

HLT

- Write a program to

  - clear the accumulator

  - add 47H (use ADI instruction)

  - subtract 92H

  - add 64H

  - display the results after subtracting 92H and after adding 64H

- Specify the register contents and the flag status as the following instructions are executed

| A | B | S | Z | CY |
| --- | --- | --- | --- | --- |
| XX | XX | 0 | 0 | 0 |

XRA A

MVI B,4AH

SUI 4FH

ANA B

HLT

- What is the output at PORT1 when the following instructions are executed?

MVI A,8FH

ADI 72H

JC DSPLAY

OUT PORT1

HLT

DSPLAY:  XRA A

OUT PORT1

HLT