Student’s Name

Professor

Course

Date

- Determine the expected future population.

The future population is an important factor to consider while designing the domestic water supply.

Using Geometric -increase model, the following formula is effective in determining the population increase at the end of design period (future population).

P = P0 (1+r)n

P0 = Current population

n = design period in years

r = rate of population growth

P = Future population (population at the end of design period)

- Determine the future demand (L/s).

The future demand will be the total contribution of domestic demand, commercial and industrial demand, fire demand, and leakage losses.

The equation below gives an estimation of the total domestic demand;

Qdomestic = Qavg x p

Qavg= Average consumption per person

P= future population

Commercial and Industrial demand

This is the amount of water needed for offices, stores, and factories.

It depends on the industrialization of a particular city; hence the value should be studied separately for each case.

However, it is sometimes expressed in terms of the percentage of the domestic demand.

Agricultural demand

The amount of water for agricultural demand depends on the climate, type of soil and crop.

Fire demand

The supply network should spare a certain amount of water for fire protection.

Fire demand formulas

QF =65√P(1-0.01√P)               QF = Fire demand l/s

P = population in thousands

QF= 53√P                QF = Fire demand l/s

P = population in thousands

QF=360XC√A                             QF=fire demand flow m3 /d

A = areas of all stories of the building under consideration (m2 )

C = constant depending on the type of construction;

Leakage and Losses

It is the wastage attributable to, unauthorized connections

Errors in meter readings

Leak in the distribution system

- Determine the size of the watermain required for the expected future population (C=110). Roundup to the nearest pipe size.

Given that;

C=110

QF=360XC√A   OR   QF= 53√P    OR   QF =65√P(1-0.01√P)

360X 110√A   =  53√P

39600√A =53√P  ……….(I)

65√P(1-0.01√P)=  360X 110√A

√P(1-0.01√P) =609.23√A ……..(II)

Frrom eqn (i) √P=747.17√A

Therefore; √A=0.001338√P

Substituting in equation (II) ,

√P(1-0.01√P)= 747.17(0.001338√P)

√P(1-0.01√P)=0.9997√P

Divididing both sides by√P

1-0.01P=0.9997

-0.01P=-0.000287

P=0.028654

P=0.1693

QF= 53√P

QF=53X √0.1693

QF=21.81m3|s

Assuming a unit length pipe of 1m

The diameter of the main pipe;

QF=AxL

21.81m3|s =AXL

21.81=AX1

A=21.81

A=πr2

21.81= πr2

r2=6.942

r=2.63m

Therefore; radius of the water main pipe=2.63m

Diameter=5.27m

- What is the starting pressure at the source if the residual pressure at the town is 360 kPa? (Note: 1 m = 9.81 kpa)

Pressure= ForceArea

At the particular town, P=360kPa

From the source, the town is at, 360 kPa	=36.70m

9.81

QF=21.81m3|s

QF at the town= 21.81x36.70

=800.427kPa

e) What is the required projected fire flow? Assume fire flow demand (L/s) is calculated as: F.F. = 65√P * (1 - 0.01 √P )

F.F. = 65√P * (1 - 0.01 √P )

From part (c), P=0.1693

Therefore, F.F= 65√P * (1 - 0.01 √P )

=65√0.1693 * (1 - 0.01 √1693 )

=2663.49M3Is

f) How much storage (m3) for fire protection is required for 10 hours of protection?

F.F =2663.49M3Is

In 10 hrs., total discharge;

2663.49M3x10x60x60

=95885668.06M3