Routing and Switching Concepts

INSTITUTION_NAME

STUDENT_NAME

Introduction:

A router uses a routing table to figure out where packets are to be delivered. A list of routes identifying the gateway or protocol that the router uses to connect to a given network can be found in the table. At first only directly connected networks are in the routing table. Routes must be defined and added to the routing table to connect with remote networks.

In this study, a permanent path to a given distant network will be designed manually based on an IP address or exit interface in the next hop. A fixed standard path will also be configured. A default is a static route type, where the router table does not contain a destination network route, which specifies the gateway to be used.

Cisco 1941 Integrated Services (ISR) routers with Cisco IOS version of 15.2(4)M3 (universal 9 picture) are the routers that are used for CCNA hands-on test. The Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) are the switches used (labasek9 picture). It may also be possible to use other routers, switches or Cisco IOS.

The commands that are available and the production generated can vary according to the design and Cisco IOS version of the laboratories. For the appropriate device description please see the router interface overview table at the end of this section.

IPV4 IP Addressing

| Device | Interface | IP Addressing | Subnet Mask | Default gateway | Hosts |
| --- | --- | --- | --- | --- | --- |
| WATFORD_1 | FastEthernet0/0 | 172.26.8.1/24 00D0.D32D.546A | 255.255.255.0 | N/A |  |
|  | FastEthernet1/0 | 172.26.8.1/24 00E0.A313.1C14 | 255.255.255.0 | N/A |  |
|  | Serial2/0 | 192.168.2.1/30 000D.BD3B.586D | 255.255.255.252 | N/A |  |
|  | FastEthernet4/0 | 172.17.8.1/30 0007.EC05.C3C7 | 255.255.255.0 | N/A |  |
| WATFORD_2 | FastEthernet0/0 | 172.18.8.1/24 00D0.BA16.A039 | 255.255.255.0 | N/A |  |
|  | FastEthernet1/0 | 172.19.8.1/24 000B.BE4B.6AE5 | 255.255.255.0 | N/A |  |
|  | FastEthernet4/0 | 172.17.8.2/24 00E0.F916.27D0 | 255.255.255.0 | N/A |  |
| STEVENAGE_1 | FastEthernet0/0 | 10.0.0.1/30 0001.63AC.610E | 255.255.255.0 | N/A |  |
|  | Serial3/0 | 10.0.0.1/30 0010.11DA.D85D | 255.255.255.0 | N/A |  |
|  | FastEthernet4/0 | 10.0.2.1 /30 0001.4381.5E45 | 255.255.255.0 | N/A |  |
| STEVENAGE_2 | FastEthernet0/0 | 10.0.3.1 /30 0001.97A9.8700 | 255.255.255.0 | N/A |  |
|  | FastEthernet4/0 | 10.0.2.2 /30 0060.3E73.0918 | 255.255.255.0 | N/A |  |
| STEVENAGE_3 | FastEthernet0/0 | 10.0.5.1 /30 0002.4A3D.E53D | 255.255.255.0 | N/A |  |
|  | FastEthernet5/0 | 10.0.4.2/30 0090.0C37.3375 | 255.255.255.0 | N/A |  |
| ISP | FastEthernet0/0 | 192.168.3.1 /24 00D0.BA16.A039 | 255.255.255.252 | N/A |  |
|  | Serial2/0 | 192.168.2.2 /24 000B.BE4B.6AE5 | 255.255.255.252 | N/A |  |
| SERVER_2 | DNS | 8.8.8.8 | N/A | N/A |  |
|  | FastEthernet0/0 | 192.168.3.2 /24 | 255.255.255.252 | 192.168.3.1 | 1 |
| SERVER_0 | DNS | 10.0.4.10 | N/A | N/A |  |
|  | FastEthernet0/0 | 10.0.4.10 /30 | 255.255.255.0 | 10.0.4.2 | 1 |
| SERVER_1 | DNS | 10.0.4.11 | N/A | N/A |  |
|  | FastEthernet0/0 | 10.0.4.11 /30 | 255.255.255.0 | 10.0.4.2 | 1 |
|  |  | MANAGEMENT |  |  | 25 |
| PC_4 | FastEthernet0/0 | 172.26.8.10 /24 | 255.255.255.0 | 172.26.8.1 |  |
| PC_5 | FastEthernet0/0 | 172.26.8.11 /24 | 255.255.255.0 | 172.26.8.1 |  |
|  |  | ENGINEERING |  |  | 200 |
| PC_6 | FastEthernet0/0 | 172.16.8.10 / 24 | 255.255.255.0 | 172.16.8.1 |  |
| PC_7 | FastEthernet0/0 | 172.16.8.11 /24 | 255.255.255.0 | 172.16.8.1 |  |
|  |  | RESEARCH |  |  | 100 |
| PC_8 | FastEthernet0/0 | 172.18.8.10 / 24 | 255.255.255.0 | 172.18.8.1 |  |
| PC_9 | FastEthernet0/0 | 172.18.8.11 /24 | 255.255.255.0 | 172.18.8.1 |  |
|  |  | SALES |  |  | 200 |
| PC_10 | FastEthernet0/0 | 172.19.8.10 /24 | 255.255.255.0 | 172.19.8.1 |  |
| PC_11 | FastEthernet0/0 | 172.19.8.11 /24 | 255.255.255.0 | 172.19.8.1 |  |
|  |  | MARKETING |  |  | 50 |
| PC_0 | FastEthernet0/0 | 10.0.0.10 /30 | 255.255.255.0 | 10.0.0.1 |  |
| PC_1 | FastEthernet0/0 | 10.0.0.11 /30 | 255.255.255.0 | 10.0.0.1 |  |
|  |  | HR |  |  | 10 |
| PC_2 | FastEthernet0/0 | 10.0.3.10 /30 | 255.255.255.0 | 10.0.3.1 |  |
| PC_3 | FastEthernet0/0 | 10.0.3.11 /30 | 255.255.255.0 | 10.0.3.1 |  |

Ping from PC_1 to PC_2, this shows the communication between two PC’S located in 2 different network segments.

Ping from PC 1 to WATFORD_1, this shows the communication of the PC and the FastEthernet0/0 of the Router, both addresses are on the same network range.

Full Router configuration showed with the command Show Running Configuration or (show run)

WATFORD_1(config)#do show run

building configuration…

Current configuration : 993 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname WATFORD_1

!

enable secret 5 $1$mERr$9cTjUIEqNGurQiFU.ZeCi1

!

!

no ip domain-lookup

ip PC WATFORD_1 192.168.0.1

!

!

interface FastEthernet0/0

description WATFORD_1 Fa0/0

ip address 192.168.1.1 255.255.255.0

duplex auto

speed auto

!

interface FastEthernet1/0

no ip address

duplex auto

speed auto

shutdown

!

interface Serial2/0

description WATFORD_1 Serial 2/0

ip address 192.168.0.1 255.255.255.0

clock rate 64000

!

interface Serial3/0

no ip address shutdown

!

interface FastEthernet4/0

no ip address shutdown

!

interface FastEthernet5/0

no ip address

shutdown

!

router rip

version 2

network 192.168.0.0

network 192.168.1.0

!

ip classless

!

!

Banner motd ^C

Keep away from this network!

^C

!

line con 0

password cisco

login

line vty 0 4

password class

login

line vty 5

password class login

!

end

The screenshot below shows a telnet connection from the PC computer that is located in a different network range. Telnet enables remote connection into Routers and Switches for configuration and maintenance. Telnet is not always available in companies that are highly concerned of network security because it opens a port for external connection.

Tracert find out how many hopes away a PC is from the gateway of the service it’s trying to reach, tracert www.t2electro.co.uk display all the Routers that will be by your packet. In my case I have used tracert 192.168.0.8 that shows 1 hop, which means that I am 1 hop away from my gateway.

IP address of the router interfaces

router 0

Router(config)#in fa 0/0
Router(config-if)#ip add 192.168.1.1 255.255.255.0
Router(config-if)#no sh

router 1

Router(config)#in fa 0/0
Router(config-if)#ip add 192.168.1.2 255.255.255.0
Router(config-if)#no sh

Router(config-if)#in se 2/0
Router(config-if)#ip add 192.168.2.1 255.255.255.0
Router(config-if)#no sh
Router(config-if)#clock rate 56000

router 2

Router(config)#in se 2/0
Router(config-if)#ip add 192.168.2.2 255.255.255.0
Router(config-if)# no sh

rip protocol and disable auto summarization

router 0

Router(config)#router rip
Router(config-router)#ver 2
Router(config-router)#network 192.168.1.0
Router(config-router)#no auto-summary

Router 1

Router(config)#router rip
Router(config-router)#ver 2
Router(config-router)#network 192.168.1.0
Router(config-router)#network 192.168.2.0
Router(config-router)#no auto-summary

router 2

Router(config)#router rip
Router(config-router)#ver 2
Router(config-router)#network 192.168.2.0
Router(config-router)#no auto-summary

Router(config)#interface fastEthernet 0/0

Router(config-if)#ip address 192.168.1.1 255.255.255.0

Router(config-if)#no shutdown

router 0 name as arizona, router 1 as virginia, router 0

Router(config)#PCname arizona

router 1

Router(config)#PCname virginia

router 2

Router(config)#PCname NY

describtion of the interfaces

router 0

Router(config)#in fa 0/0

Router(config-if)#description connection to headquater

router 1

Router(config)#in fa 0/0

Router(config-if)#description connection to branch 1

Router(config)#in se 2/0

Router(config-if)#description connection to branch 2

router 2

Router(config)#in se 2/0

Router(config-if)#description connection to headquater

MOTD on all routers to ---- UNAUTHORIZED ACCESS IS NOT ALLOWED ----

On all routers

Router(config)#banner motd / ---- UNAUTHORIZED ACCESS IS NOT ALLOWED ---- /

References:

Mir, N. (2019). 1.5 Addressing Scheme in the Internet | Introduction to Packet-Switched Networks | InformIT. [online] Informit.com. Available at: http://www.informit.com/articles/article.aspx?p=2272153&seqNum=5 [Accessed 10 Nov. 2019].

Wilkins, S. (2019). Creating a Basic IP Addressing Scheme | Creating a Basic IP Addressing Scheme | Pearson IT Certification. [online] Pearsonitcertification.com. Available at: http://www.pearsonitcertification.com/articles/article.aspx?p=1843889 [Accessed 10 Nov. 2019].

Freeccnaworkbook.com. (2019). Configuring Routing Information Protocol (RIP) | Free CCNA Workbook. [online] Available at: https://www.freeccnaworkbook.com/workbooks/ccna/configuring-routing-information-protocol-rip [Accessed 10 Nov. 2019].