Instrument height = Back sight elevation + elevation

Elevation = Instrument height – Foresight

FIRST CASE

| Station | Foresight | Backsight | Instrument height | Elevation |
| --- | --- | --- | --- | --- |
| BM A |  | 6.94 | 665.53 | 658.60 |
| 1ST INST |  |  | 665.53 |  |
| TP#1 | 3.67 | 5.17 | 664.01 | 658.84 |
| 2ND INST |  |  | 664.01 |  |
| TP#2 | 8.81 | 7.75 | 665.95 | 658.20 |
| 3RD INST |  |  | 665.95 |  |
| TBM 1 | 10.34 |  | 655.65 |  |

BS –FS = LAST POINT ELAVATION – FIRST POINT ELEVATION

6.94 + 5.14 + 7.75 – 3.64 =655.61 – 658.60

SECOND CASE

| Station | Foresight | Backsight | Instrument height | Elevation |
| --- | --- | --- | --- | --- |
| BM A |  | 5.42 | 507.55 | 502.13 |
| 1ST INST |  |  | 507.55 |  |
| TP#1 | 9.01 | 7.55 | 507.11 | 498.56 |
| 2ND INST |  |  | 507.11 |  |
| TP#3 | 3.61 | 4.99 | 507.46 | 502.47 |
| 3RD INST |  |  | 507.46 |  |
| TP#3 | 8.22 | 6.83 | 506.08 | 499.25 |
| 4TH INST |  |  | 506.08 |  |
| TBM 1 | 3.37 |  |  | 502.71 |

To avoid errors in the calculations, the heights of the beck sight and the foresight should not exceed 150ft. The instrument's height is given by elevation added to the backsight, where elevation is the instrument height less the foresight. Conversely, the difference in sights equals the elevation difference.