Selected Topics Course (Communication Lab)

# Abstract

The purpose of this report is to reflect theoretical and practical knowledge gained from a Master course, "Selected Topics in Communication Lab." This educational course/lab was designed to cover many features and topics that weren't practically clear/understandable from previous theoretical courses. The report's theoretical part in this report only explains and supports students to get the highest quality of knowledge from this course. The practical information part is reflecting the trainee's results from each experiment. Moreover, the Lab contains software and valuable equipment that delivers an advanced knowledge level in Communication Systems.

Table of Contents

Abstract	2

Table of contests	3

Communication Systems	4

Frequency Ranges	5

LAB Transmitter Board and Another Measurement Equipment	5

Amplitude Modulation (AM)	7

Generation of DSB-AM	7

Frequency Modulation (FM)	13

Amplitude-shift keying (ASK)	16

Phase shift keying (PSK)	17

Frequency shift keying (FSK)	18

Quadrature amplitude modulation (QAM)	20

Communication Systems

In communication, modulation is defined as the process by which some characteristic of a carrier wave is varied following an information-bearing signal. The carrier is needed to facilitate the modulated signal's transportation across a band-pass channel from the transmitter to the receiver.

We are starting from the speaker, the voice signal passing through the transducer. Then we can get an electrical signal. The electrical signal is processing across an electrical processor. After that, it will be ready for transmitting as an electromagnetic wave signal. Therefore, the electromagnetic signal would reach a receiver that translating the electromagnetic signal to an electronic signal. Across the transducer, the electronic password will be changed to be a familiar voice signal and ready for the ear.

Frequency Ranges

Due to their various attributes and depending on the application, electromagnetic waves of different wavelengths are used for message transmission. The usable frequency range stretches from a few kHz to many GHz.

Therefore, the radio wave is an electromagnetic wave of a frequency between about 104 and 1011 or 1012 Hz, as used for long-distance communication.

LAB Transmitter Board and another Measurement Equipment

At the LAB, we have many components that support us in understanding the different telecommunication technologies.

One of the main components at the LAB is the experiment board, which is broken down into various functional units.

So, we started the first experiment at this course using a simple transmission way.

1.	We connected a mobile phone with a board through Bluetooth technology.

2.	Through the experiment board, the signal was amplitudes.

3.	After that, the signal is transmitted through the antenna (wireless communication).

4.		So, the 2nd experiment board received the password through another antenna (receiver).

5.	The 2nd board has a decoder for decoding the signal and making it normal to be presented through the speaker.

The following are some of the most essential pieces of measurement equipment used in transmission technology:

- Oscilloscope,

- Frequency counter,

- Spectrum analyzer

- Network analyzer

- Reflect meter

- SWR meter.

Amplitude Modulation (AM)

In the communication world, modulation is defined as how some characteristics of a carrier wave are varied following an information-bearing signal. To simplify AM's meaning AM is a modulation technique used in electronic communication, most commonly for transmitting messages with a radio carrier wave. In amplitude modulation, the amplitude (signal strength) of the carrier wave is varied in proportion to that of the message signal, such as an audio signal.

AM was the earliest modulation method used for transmitting audio in radio broadcasting. It was developed during the first quarter of the 20th century, beginning with Roberto Landell De Moura and Reginald Fessenden's radiotelephone experiments in 1900.

Generation of DSB-AM

In the following experiment, the multiplication of 2 oscillations is investigated in the time and frequency domain.

- Carrier 10 kHz.

- Variable sine.

- Wave from the function generator.

- Connect the function generator and the oscilloscope following the circuit diagram. Set the function generator to DC, 1 V.

- Open the oscilloscope and measure the amplitudes of Y1 and Y2. Calculate the modulation coefficients kM and enter the result into the appropriate field as the modulator's left input is biased with 1 V DC.

- Set the function generator to: Sine, 200 Hz, 10 Vpp. Open the oscilloscope and adjust the settings to view the curve (X/div = 0.5 ms).

- Increase the function generator's frequency up to 4 kHz in several steps and observe the changes in the curve displayed on the oscilloscope.

Frequency (780 Hz)

Frequency (4000 Hz)

- Next, set the function generator's frequency to 1 kHz and change the signal shape one after the other to Triangular and Square-wave.

- Remove the connection to the function generator and use a connecting lead to connect the Carrier socket to the modulator's input. Observe the signal first with the oscilloscope and then the FFT module.

- Furthermore, connect the circuit as shown in the circuit diagram—AM transmitted carrier (full AM).

- Set the function generator to Sine, 1 kHz, 10 Vpp, DC = 0 V. Open the oscilloscope and adjust it so that the curve characteristic is displayed (X/div = 0.5 ms).

- Randomly vary the function generator's frequency setting between 500 Hz and 2000 Hz and set the amplitude between 0 Vpp and 10 Vpp.

F=500Hz, V=10 Vpp

F=1500Hz, V=10 Vpp

F=2000Hz, V=10 Vpp

F=1000Hz, V=5 Vpp

F=1000Hz, V=2 Vpp

F=1000Hz, V=2 Vpp

- As shown in the circuit diagram, remove the carrier's connection and the summation stage's upper input. The carrier should now disappear from the spectrum. Slowly increase the DC component to approx. DC = 2 V on the function generator. As a result, the carrier reappears in the output signal.

Frequency Modulation (FM)

It is the encoding of information in a carrier wave by varying the instantaneous frequency of the wave. The technology is used in telecommunications, radio broadcasting, and signal processing. Frequency modulation (FM) and phase modulation (PM) form the angular modulation methods. Frequently FM modulators are designed as voltage-controlled oscillators or VCOs.

In this experiment, the static characteristic of the VCO is recorded. This depends on the middle frequency presetting using Frequency. Using the Amplitude controller, the amplitude of the VCO output signal can be set.

- Connect the function generator and the oscilloscope to the experiment board, according to the Figure above. Set the function generator initially to DC = 0 V.

- Open the frequency counter. In the operating panel of the frequency counter, set the Trigger to +Y2. Turn the Amplitude potentiometer to the far-right stop (max). Set the Frequency potentiometer so that there is a frequency of approx—10 kHz at the output.

- Set a DC voltage of -10 V on the function generator. Open the characteristics plotter and enter the value pair (U, f) into its Table. Increase the voltage at the VCO input in steps of 2.5 V until you reach 10 V and enter the measured value pairs into the Table. Then set the voltage at the information to 0 V.

- Determine the modulation coefficient k in the region around Uin = 0 V based on the characteristic slope. Use the markers of the characteristics plotter to accomplish this task. Enter the result in the appropriate field. Make sure you note down the correct sign. K=-282.8.

Another experiment was done related to the FM signal.

- Connect the function generator and the oscilloscope to the experiment board according to the above illustration. Set the function generator initially to DC = 0 V.

- Besides the oscilloscope, open the FFT module and set it to Y1, f max=20 kHz. Start the measurement.

Input voltage=0 v DC

Amplitude-shift keying (ASK)

It is a form of amplitude modulation representing digital data as variations in a carrier wave's amplitude. In an ASK system, the binary symbol is represented by transmitting a fixed-amplitude carrier wave and fixed Frequency for a bit of T seconds. If the signal value is 1, then the carrier signal will be mailed; otherwise, a signal value of 0 will be sent.

ASK diagram

ASK system can be divided into three blocks. The first one represents the transmitter; the second one is a linear model of the channel's effects; the third one shows the receiver's structure. The following notation is used:

- ht(f) is the carrier signal for the transmission

- hc(f) is the impulse response of the channel

- n(t) is the noise introduced by the channel

- hr(f) is the filter at the receiver

- L is the number of levels that are used for transmission

- Ts is the time between the generation of two symbols

Phase shift keying (PSK)

It is a digital modulation process that conveys data by changing (modulating) the constant frequency reference signal (the carrier wave). The modulation is accomplished by varying the sine and cosine inputs at a precise time. It is widely used for wireless LANs, RFID, and Bluetooth communication.

In addition to the advantages found in FSK, PSK also benefits from a higher immunity to interference. In the experiment, the AM modulator's PSK signal is produced in conjunction with a pure AC signal. The spectrum of the PSK corresponds to that of a DSB-AM call without a carrier.

- Connect the function generator to the oscilloscope, as specified in the circuit diagram.

- Set the function generator to curve shape square-wave, frequency 1kHz, amplitude 4 Vpp, 0 V DC, pulse duty cycle 50%.

- Open the oscilloscope and make the following settings: Curve Dual, X/div 100 μs, Y1/div = Y2/div = 5 V, Trigger Off. Start the measurement.

- Also, open the FFT module and set it to Y2, fax = 20 kHz and then start the measurement.

Frequency shift keying (FSK)

It is a Frequency modulation scheme in which digital information is transmitted through discrete frequency changes of a carrier signal. The (binary) FSK transmits data at the discrete frequencies f1 and f2. Any lack of transmission signal must automatically be attributed to faulty transmission. FSK represents a particular case of FM transmission. As such, it has a similarly expanded and complicated spectrum. FSK is the preferred form of data transmission via the telephone system using modems. Regarding modems, a distinction is drawn between the following operating modes:

- Simplex

- Duplex

- Half-duplex

The simplest FSK is binary FSK (BFSK). BFSK uses a couple of discrete frequencies to transmit binary (0s and 1s) information.

An example of binary FSK

A binary FSK signal's demodulation can be done using the Goertzel algorithm very efficiently, even on low-power microcontrollers. The Goertzel algorithm is a digital signal processing (DSP) for efficient evaluations of the individual terms of the discrete Fourier transform (DFT). It is

Useful in specific practical applications, such as recognizing dual-tone multi-frequency signaling (DTMF) tones produced by the keypad's push buttons of a traditional analog telephone.

- In the experiment, connect the function generator G1 and the oscilloscope to the experiment board, as shown in the illustration above. First, set the function generator to DC = 0 V.

- Open the frequency counter. In the operating panel of the frequency counter, set Trigger to Y2. Turn the Amplitude potentiometer to the right far limit. Set the Frequency potentiometer so that a frequency of approx. 10 kHz is applied to the output.

- Set the function generator as follows: square-wave, 1 kHz, 20 Vpp, DC = 0

V. Open the oscilloscope and set it to: X/div = 200 μs, Y1/div = Y2/div = 5 V, Curve Dual, and Trigger Off. Start the measurement.

- In addition to the oscilloscope, open the FFT module, set it to Y2, f max = 20 kHz, and then start the measurement.

Quadrature amplitude modulation (QAM)

The family of digital modulation methods and a related family of analog modulation methods widely used in modern telecommunications to transmit information. It conveys two analog message signals, or two digital bit streams, by changing (modulating) the amplitudes of two carrier waves, using the amplitude-shift keying (ASK) digital modulation scheme or amplitude modulation (AM) analog modulation scheme. The two carrier waves of the same frequency are out of phase by 90°, a condition known as orthogonality or quadrature. The transmitted signal is created by adding the two carrier waves together. The two waves can be coherently separated (demodulated) because of their orthogonal property. Another fundamental property is that the modulations are low-frequency/low-bandwidth waveforms compared to the carrier frequency, known as the narrowband assumption.

QAM is used extensively as a modulation scheme for digital telecommunication systems, such as 802.11 Wi-Fi standards. High spectral efficiencies can be achieved with QAM by setting a suitable constellation size, limited only by the communications channel's noise level and linearity. QAM is being used in optical fiber systems as bit rates increase; QAM-16 and QAM-64 are evolutions of the QAM family.

Bit sequence mapping for a 16QAM signal

Digital 16-QAM with example constellation points