Laboratory Report No. ?

Friction

Date

Your Name???

Course name and number

Objective

To measure the coefficients of friction: static friction and kinetic friction.

Equipment

- Wood plank

- Mass blocks

  - Cork

  - Felt

  - Plastic

- Mass hanger

Theory

Set-up:

m = mass of block (including any added mass)

M = mass of hanger (includes hanger itself and any added mass)

We have T=MHg and n=mg.

If we are near (and slightly above fs,max, the block will begin to move on its own so, fs,max ≅ T  μsn = MHg  μsmg = MHg.

Procedure

- Measure μs and μk between the wood plank and the block.

- Do this for each of three block types.

- Repeat each scenario for different values of added mass (Choose from +250g, +500g, +750g).

- Produce plots of μs vs k and μk vs k for each block type.

Data and Observations

The data appears to be accurate. The material having most friction is the cork while the least is the plastic. Static friction is greater than the kinetic friction.

Data Analysis

Normal force=Mass of material×Gravitational constant

Normal force=0.281kg×9.8 m/s2

Normal force=2.78

μs=NFs=2.78 N0.69

μs=0.248

μk=NFk=2.78 N0.49

μk=00.176

Repeat the calculations for all trials for all materials considered.

Results

| Item | Mass (kg) | Normal Force (N) | Fs (N) static friction | Fk (N) kinetic friction | μs coefficient of static friction | μk coefficient of kinetic friction |
| --- | --- | --- | --- | --- | --- | --- |
| Felt (0.2 kg added) | 0.281 | 2.78 | 0.69 | 0.49 | 0.248201439 | 0.176258993 |
| Felt (0.4 kg added) | 0.481 | 4.74 | 0.9 | 0.76 | 0.189873418 | 0.160337553 |
| Felt (0.6 kg added) | 0.681 | 6.71 | 1.44 | 1.08 | 0.214605067 | 0.1609538 |
| Item | Mass (kg) | Normal Force (N) | Fs (N) static friction | Fk (N) kinetic friction | μs coefficient of static friction | μk coefficient kinetic friction |
| Cork (0.2 kg added) | 0.29 | 2.83 | 1.11 | 0.9 | 0.392226148 | 0.318021201 |
| Cork (0.4 kg added) | 0.49 | 4.83 | 1.85 | 1.5 | 0.383022774 | 0.310559006 |
| Cork (0.6 kg added) | 0.69 | 6.8 | 2.77 | 2.29 | 0.407352941 | 0.336764706 |
| Item | Mass (kg) | Normal Force (N) | Fs (N) static friction | Fk (N) kinetic friction | μs coefficient of static friction | μk coefficient kinetic friction |
| Plastic (0.2 kg added) | 0.2935 | 2.88 | 0.65 | 0.57 | 0.225984772 | 0.198171262 |
| Plastic (0.4 kg added) | 0.4935 | 4.84 | 1.15 | 0.85 | 0.237785084 | 0.175754192 |
| Plastic (0.6 kg added) | 0.6935 | 6.80 | 1.54 | 1.28 | 0.226593882 | 0.188337772 |

Discussion/Conclusion

The coefficient of friction is different for different materials. The cork has the highest coefficient of friction, then felt and plastic has the least. The more mass a material has, the more normal force it experiences. However, the coefficient of friction remains almost constant given a good set-up of experiment.