Center of Pressure and Hydrostatic Force On Submerged Bodies

Name:

ID:

Abstract

This lab report fundamentally presents the results  and analysis of the Center of Pressure and Hydrostatic Force On Submerged Bodies experiment with the aim of hydrostatic pressure distribution, calculate the magnitude of the hydrostatic force on a submerged body, and calculate and verify the location of center of pressure. Additionally the results and discussion on the experiments are tabled calculations done to determine hydrostatic force and hypothetical  relations established.

3. Table of contents

Table of Contents Abstract......................................................................................................................................2

List of figures.............................................................................................................................4

List of tables...............................................................................................................................5 Introduction:...............................................................................................................................6 Theory:.......................................................................................................................................6 Procedure: ..................................................................................................................................9

Result and Discussions ..............................................................................................................11

Sources of Errors: ....................................................................................................................14

Conclusion: ..............................................................................................................................15

References:...............................................................................................................................17

4. List of figures

Figure 1: experiment setup with labels of apparatus    ……………………………5

Figure 2: Tilted plate……………………………………………………………………………….6

Figure3: vertical plate……………………………………………………………………………….6

List of tables

Table 1: lab results…………………………………………………………………………………………………9

Introduction

In this experiment, the hydrostatic pressure distribution is measured and observed, as well as the magnitude of the hydrostatic force on a submerged body. Lastly, we calculate and verify the location of center of pressure. In the results analysis, the experimentally determined values will be compared to theoretical calculation.

Figure 1: experiment setup with labels of apparatus

Theory

When a rectangular plate with height b and width a, is completely submerged and tilted an angle from the horizontal surface with an horizontal top edge and at a distance s from the free surface as shown below.

Figure 2: Tilted plate

Figure3: vertical plate

The resultant hydrostatic force on the upper surface is equal to the average pressure, which is the pressure at the midpoint of the surface, times the surface area A. This is to say that;

𝐹𝑅 = 𝑃𝑐𝐴 = [𝑃𝑜 +𝜌𝑔 (𝑠 + 𝑏 2 ) 𝑠𝑖𝑛𝜃] 𝑎𝑏------------- equation 1

𝑦𝑝 = 𝑦𝑐 + 𝐼𝑥𝑥,𝑐 /[𝑦𝑐 + 𝑃𝑜 𝜌𝑔sin𝜃 ] 𝐴-----------------equation 2

Directly beneath the centraoid of the plate, the force acts at a vertical distance of hp=ypsin θ. At this point equation 2 translates to;

𝑦𝑝 = 𝑠 + 𝑏/2 + {(𝑎𝑏3/12) /[𝑠+ 𝑏/2 + (𝑃𝑜 /𝜌𝑔 𝑠𝑖𝑛 𝜃) ]𝑎𝑏}

𝑦𝑝 = 𝑠 + 𝑏/2 + {𝑏2 /12/[𝑠+ 𝑏/2 + (𝑃𝑜/𝜌𝑔 𝑠𝑖𝑛 𝜃) ]------------- equation 3

For a plate with an horizontal edge that is completely submerged (𝜃=90 degrees), the hydrostatic force is obtained by setting sin 90deg=1 as shown above in fig. 3, is given by;

𝐹𝑅 = [𝑃𝑜 + 𝜌𝑔 (𝑠 + 𝑏/2 )] 𝑎𝑏------------------- equation 4

Procedure

During the experiment, the necessary calibrations on the rig were done and then the setup placed on an horizontal surface and slight adjustments using the adjustable lids to ensure that it was levelled. To level off the balance bridge, an empty hook was attached to a counter weight. The gate valve was then closed and the we filled the tank until the rectangular plate on the quadrant was fully submerged. The water level was higher than the height of the rectangular surface. Aimed at balancing out the balance bridge with the amount of water by weights were added to the hook and the balance bridge was restored to its balance position. The water depth was taken and recorded for use in the calculation of hydrostatic force acting on the vertical plate. A moment balance on the pivot of the moving arm was performed to determine the point of action of the hydrostatic force. The point of action of the hydrostatic force was calculated using equation 3and compared the value experimentally obtained from the previous step. The steps were repeated for different water levels.

Results and discussions

| b(m) | Mass (kg) | FR(N) | Yp,th(m) | L(m) | α (m) | Yp,Exp(m) | Error% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.1 | 0.202 | 3.4300 | 0.0667 | 0.5642 | 0.3642 | 0.4642 | 5.96 |
| 0.09 | 0.162 | 2.7783 | 0.0600 | 0.4525 | 0.2525 | 0.3425 | 4.z71 |
| 0.08 | 0.132 | 2.1952 | 0.0533 | 0.3687 | 0.1687 | 0.2487 | 3.67 |
| 0.07 | 0.102 | 1.6807 | 0.0467 | 0.2849 | 0.0849 | 0.1549 | 2.32 |
| 0.06 | 0.077 | 1.2348 | 0.0400 | 0.2151 | 0.0151 | 0.0751 | 8.78 |

Table 1: lab results

Calculations

FR in N

FR= pgab2/2

a= 0.07m, p=1000kg/m^3 p= 9.8 m/s2

At b=0.1m; FR= (1000 kg/m^3 *9.8m/s^2*0.07m *0.1^2)/2 =3.4300N

At b=0.09m; FR= (1000 kg/m^3 *9.8m/s^2*0.07m *0.09^2)/2 =2.7783N

At b=0.08m; FR= (1000 kg/m^3 *9.8m/s^2*0.07m 0.08^2)/2 =2.1952N

At b=0.07m; FR= (1000 kg/m^3 *9.8m/s^2*0.07m *0.07^2)/2 =1.6807N

At b=0.06m FR= (1000 kg/m^3 *9.8m/s^2*0.07m *0.06^2)/2 =1.2348N

Yp,th(m); y=2b/3

At b=0.1m; yp,th=2*0.1/3=0.0667m

At b=0.09m; yp,th=2*0.09/3= 0.0600m

At b=0.08m; yp,th=2*0.08/3=0.0533m

At b=0.07m; yp,th=2*0.07/3=0.0467m

At b=0.06m; yp,th=2*0.06/3=0.0400m

L(m); L=0.285mg/FR

At m=0.202kg; L=0. 285*0.202kg*9.8m/s^2= 0.5642m

At m=0.162kg; L=0. 285*0.162kg *9.8 m/s^2=0.4525m

At m=0.132kg; L=0. 285*0.132kg *9.8 m/s^2=0.3687m

At m=0.102kg; L=0. 285*0.102kg *9.8 m/s^2=0.2849m

At m =0.077kg; L=0. 285*0.077kg*9.8 m/s^2=0.2151m

α (m) = 0.2-L

For b=0.1m; α (m) = 0.2-0.5642=-0.3642m

For b=0.09m; α (m) = 0.2-0.4525 =0-0.2525m

For b=0.08m; α (m) = 0.2-0.3687 =-0.1687m

For b=0.07m; α (m) = 0.2-0.2849=0.0849m

For b=0.06m; α (m) = 0.2-0.2151=0.0151m

Yp,Exp(m)= b- α

For b=0.1m Yp,Exp =0.1--0.3642m=0.4642m

For b=0.09m Yp,Exp =0.09m--0.2525m=0.3425m

For b=0.08m Yp,Exp =0.08m--0.1687m=0.2487m

For b=0.07m Yp,Exp =0.07m-0.0849m=0.1549m

For b=0.06m Yp,Exp =0.06m-0.0151m=0.0751m

Error%= (Experimental- Theoretical)/Theoretical*100

=(0.4642m-0.0667m)/ 0.0667m*100=5.96%

=(0.3425-0.0600)/0.0600m*100=4.71%

=(0.2487-0.0533)/0.05333m*100= 3.67%

=(0.1549-0.0467)/0.0467m*100= 2.32%

=0.0751-0.0400/ 0.0751m*100=8.78%

Analysis

`

10. Sources of error

The sources  of error include inherent limitation of measuring devices used. The rulers and the initial labelling’s.

Conclusion

The hydrostatic pressure force acting on the rectangular plate. The equivalent force will act in a direction perpendicular to the surface through which it acts. By definition, the centre of pressure will lie on this equivalent forces line of action.

References

Jasim, N. A., & Shamkhi, M. S. (2020). The design of the center of pressure apparatus with demonstration. Cogent Engineering, 7(1), 1843225.