Liquid Chromatography-Mass Spectrometry

Name

Institution affiliation

Course name

Instructor

Date

Liquid Chromatography-Mass Spectrometry

Liquid Chromatography-Mass Spectrometry (LC-MS) is defined to be an analytical technique that uses high-resolution chromatographic separation, made with highly sensitive and detailed mass spectrum detection (McPolin, 2009). It includes a combination of High-Performance Liquid Chromatography (HPLC)-MS, Capillary Electro chromatography (CEC)-MS, and Capillary Electrophoresis (CE)-MS. Mass spectrometry in Liquid Chromatography-Mass Spectrometry assists in determining the composition of elements and structural interpretation of a sample (Kumar & Vijayan, 2014).

Principle of LC-MS

Basically, the LC-MS system is a combination of HPLC and MS by the use of an ionization source or interface (McPolin, 2009). LC separates the sample used. When the sample species are separated, they are sprayed into the atmospheric pressure ion source.  Here, there is the change of the sample species into ions. Ions sorting is established by a mass analyzer where a ratio of mass to charge is developed. Then, ions emerging from the mass analyzer are counted by a detector (Ho et al., 2003). There is also amplification of signal generated from every ion emerging. By this, a mass spectrum is created and used in the determination of the orisotopic nature of elements in the sample, particles, and molecules masses, as well as elucidating the chemical structures of those molecules.

The LC that is normally used in LC-MS is High-Performance Liquid Chromatography. The principle of separation in High-Performance Liquid Chromatography is a reverse stage kind of adsorption or normal stage mode. This phase constricts with a polar motionless phase, which has non-polar solvents, best known as the mobile phase (Determann, 2012). However, the technique is used and suitable for pharmaceutical applications or biomedical research because most drugs are polar.

Technical properties of LC-MS

- Ionization source- it has a variety of categories, namely, Atmospheric Pressure Chemical Ionization (APCI), Matrix-assisted laser desorption/ionization (MALDI), and electrospray ionization (ESI) (Calderaro et al. 2014).

- Electrospray ionization (ESI)- produces an electrospray. For this to happen, there must be an application of a strong electrical field onto a liquid made to pass through a duct made with a weak fluidity. This process hence leads to the production of charged large droplets, which are required to be subjected to solvent evaporation. An increased charge density leads to solvent evaporation that results in coulombic repulsion, which overcomes the liquid surface tension, after which there is a release of ions from the droplets (Ho et al., 2003).

- Atmospheric pressure chemical ionization (APCI)- There is the involvement of nebulization of the mobile phase. This is done with nitrogen gas and a form of vaporization by ensuring the process of heating is up to above 4000C (Determann, 2012). Then, the produced vapor is exposed to a corona discharge electrode in order to create ions.

- Matrix-assisted laser desorption/ionization (MALDI)- This is an ionization technique meant to produce large and labile molecules like polymers, peptides, and proteins. The procedure involves the embedding process of analytes in a matrix. In the process, there is the absorption of energy at the wavelength of the laser (Calderaro et al., 2014).

- The mass spectrometer analyzer- mass spectrometer in the analyzer takes ionized molecules, after which it separates them according to their charge to mass ratios. Then, it takes them to the detector. Here, they are detected and soon changed to digital output.

- Mass spectrometric data acquisition system- this system is designed to help in digitalizing electrical signals taken from detectors. Then, they transfer the signals to the data system in a more and suitable compatible format.

Limitations of LC-MS in biopharmaceutical analysis

The biopharmaceutical industries face issues with LC-MS, including complex sample matrices, component analysis comprising multiple targets and unknowns, trace-level analytes, and complex plus time-consuming sample preparation methods.

Applications LC-MS in biopharmaceutical analysis

- Protein digestion- biopharmaceuticals mostly prefer use of LC-MS due to its high sensitivity. It is more complex as compared to the commonly used Tandem mass spectrometry.LC-MS is basically used in the analysis, which involves enzymatic digestion of any biopharmaceutical to required mixtures of small peptides (Ho et al. 2003). Then, there is an examination of digest and quantitation signature peptides, which helps to measure intact protein.

- Signature peptide selection- When using LC-MS, the selected signature peptide should have an exceptional amino acid sequence. The sequence should not naturally occur in the available endogenous matrix proteins (Ragoussis et al. 2006). Particularly for biopharmaceuticals, it is a requirement that the selection process must be only for the unique signature peptide. Non-unique ones may result in overestimation of analyte concentrations.

- Protein extraction- This approach is commonly used for the quantitation of endogenous proteins. Examples of such include biomarkers (Lim & Lord, 2002). The approach is preferred due to well-characterized immunological components, which are widely and readily available. However, to be assured of an improved, sensitive, and selective LC-MS method, removing interfering matrix proteins before digestion is needed by applying the immunocapture (IC) technique.

- Peptide extraction- during the extraction of a peptide, removal of interfering matrix elements is necessary after digestion at the peptide level (Ragoussis et al. 2006). The extractions' accuracy and correctness are likely to be influenced by protein-protein interaction in samples, which is a common biopharmaceutical bind to its target. Again, the approach ensures extraction of a peptide from a digest of a protein-rich biological sample, for example, plasma.

Use of a second detector format

The light-scattering detection, which is a combination of size-exclusion chromatography or HPLC, is basically an effective technique used in the characterization of antibodies and proteins (Lim & Lord, 2002). Some of the applications attached with it include; determination of absolute molecular weight, the study of PEGylated proteins, protein-drug characterization, and the study of protein aggregation.

References

Calderaro, A., Arcangeletti, M. C., Rodighiero, I., Buttrini, M., Gorrini, C., Motta, F., ... & De Conto, F. (2014). Matrix-assisted laser desorption/ionization time-of-flight (MALDI-TOF) mass spectrometry applied to virus identification. Scientific reports, 4(1), 1-10.

Determann, H. (2012). Gel Chromatography Gel Filtration· Gel Permeation· Molecular Sieves: A Laboratory Handbook. Springer Science & Business Media.

Ho, C. S., Lam, C. W. K., Chan, M. H. M., Cheung, R. C. K., Law, L. K., Lit, L. C. W., ... & Tai, H. L. (2003). Electrospray ionization mass spectrometry: principles and clinical applications. The Clinical Biochemist Reviews, 24(1), 3.

Kumar, K. J., & Vijayan, V. (2014). An overview of liquid chromatography-mass spectroscopy instrumentation.

Lim, C. K., & Lord, G. (2002). Current developments in LC-MS for pharmaceutical analysis. Biological and Pharmaceutical Bulletin, 25(5), 547-557.

McPolin, O. (2009). An introduction to HPLC for pharmaceutical analysis. Lulu. com.

Ragoussis, J., Elvidge, G. P., Kaur, K., & Colella, S. (2006). Matrix-assisted laser desorption/ionization, time-of-flight mass spectrometry in genomics research. PLoS Genet, 2(7), e100.