MGSC 368v10

Student’s Name

Course title

Date

Introduction to Production and Operations Management

Part I: Multiple Choice 								(8 Marks)

Indicate the correct answer for each of the multiple choice questions below. (2 marks each)

- A decoupling point is where inventory is carried and allows the “upstream” part of the supply chain to operate

- relatively dependent on the “downstream” part.

- Relatively independent of the “downstream” part [Correct  Answer]

- relatively independent of operational needs.

- none of the above

- For a car manufacturer, the demand for transmissions is considered

- Dependent demand.

- Independent demand.

- Decoupling  [Correct  Answer]

- Inventory redundancy.

- Four jobs must be processed through two machines. Each job must be processed on machine #1 before it can processed on machine #2. The following table gives the processing times for the jobs in hours:

| Job | Machine 1 | Machine 2 |
| --- | --- | --- |
| A | 4 | 12 |
| B | 14 | 10 |
| C | 20 | 16 |
| D | 14 | 9 |

Using Johnson’s rule, in which order should the jobs be scheduled?

- A – C – B – D (Correct  Answer)

- A – D – B – C

- D – B – C – A

- A – B – D – C

- Four Jobs must be processed through a single workcentre. The following table gives the processing times and due dates of the jobs:

| Job | Processing Time (in Days) | Due Date |
| --- | --- | --- |
| A | 5 | 9 |
| B | 10 | 7 |
| C | 4 | 11 |
| D | 6 | 6 |

Using the Critical Ratio Rule, in which order should the jobs be scheduled?

- C – A – B – D

- C – A – D – B

- D – B – C – A

- B – D – A – C [Correct  Answer]

Part II: Problems/Discussion Questions						(92 Marks)

- The Outdoor Company estimated that they will require 14,000 boxes of hiking supplies over the next year. Administrative and other costs associated with placing an order are estimated to be $23 per order. The Hiking Company uses a rate of 12% per year for calculating inventory holding costs.						(16 Marks)

The Outdoor Company’s supplier encourages its customers to order in large quantities by offering a price break on volume. Hence, the price per box depends on the order quantity as follows:

| Order Size | Price per Box |
| --- | --- |
| 0 – 349 | 32 |
| 350 – 649 | 30 |
| 650+ | 28 |

- Determine the optimal order quantity for this item.

Solution

Given that:

Annual Demand (D) = 14000 boxes

Ordering Cost (S) = $23

Holding Cost (I) = 12%

Unit Cost (C) =

- If 0 - 349 = $32

- If 350 - 649 = $30

- Above 650 = $28

Holding Cost (H) = I x C

- If 0 - 349 = 12% x 32 = 3.84

- If 350 - 649 = 12% x 30 = 3.5999 = 3.60

- Above 650 = 12% x 28 = 3.36

a. Determine the optimal order quantity for this item.

Economic Order Quantity EOQ is given by the formula:

EOQ = Sqrt( 2 x D x S / H )

Therefore:

For: 0 - 349 Range

EOQ = Sqrt( 2 x 14000 x 23 / 3.84 )

EOQ = Sqrt( 167708.33 )

EOQ = 409.52

The EOQ: 409.52 is not feasible since it exceeds the 0 - 349 range.

For: 350 - 679 Range

EOQ = Sqrt( 2 x 14000 x 23 / 3.60 )

EOQ = Sqrt( 178888.89 )

EOQ = 422.95

The EOQ: 422.95 falls within the range of 350 - 649 and hence EOQ = 422.95 is optimal.

For: Above 650+ Range

EOQ = Sqrt( 2 x 14000 x 23 / 3.36 )

EOQ = Sqrt( 191666.67 )

EOQ = 437.80

The EOQ: 437.80  is not feasible since it doesn't fall in 650+ range.

Therefore, Optimal EOQ is 422.95 = 423

- On average, how many orders will The Outdoor Company place per year?

Solution

Number of orders per year, is given by the formula:

Number of orders per year = Annual Demand (D) / EOQ

Here:

Annual Demand (D) = 14000

EOQ = 422.95 [it is a feasible EOQ which falls within the range of 350 - 649]

Therefore:

Number of orders per year = 14000 / 422.95

Number of orders per year = 33.1008 = 33

- If the lead-time is two weeks, what is the reorder point? (Assume constant demand through the year.)

Solution

Reorder Point (ROP) is given by the formula:

ROP = (Annual Demand (D) / Number of working days) x (Lead Time (Lt))

Here,

Annual Demand (D) = 14000

Lead Time (Lt) = 2 weeks

Number of working days = 52 weeks (We assume, one year = 52 weeks)

Therefore:

ROP = (14000 / 52) x 2

ROP = 538.4615 = 538

- The Furniture Company produces two products, product A and product B. Product A is made of two units of C and two units of D. Product B is made of one unit of E and one unit of F. Component F is made of two units of D and one unit of G. The gross requirements for A and B are given below:					(20 Marks)

| Week | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- |
| Product A | 100 | 80 |  | 60 | 60 |
| Product B |  | 50 | 40 | 80 |  |

- Draw a product structure tree for products A and B.

Solution

Product structure of A

Product structure of B

- Develop the MRP schedule for all items given the following information:

| Item | Present on hand | Lead time | Lot-size rule |
| --- | --- | --- | --- |
| A | 0 | 1 week | L4L |
| B | 0 | 1 week | L4L |
| C | 30 | 2 weeks | *FOQ of 40 units |
| D | 100 | 1 week | L4L |
| E | 100 | 1 week | L4L |
| F | 30 | 2 weeks | *FOQ of 60 units |
| G | 0 | 1 week | L4L |

Solution

| Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Gross req'ts |  |  |  | 100 | 80 |  | 60 | 60 |
| On hand |  |  |  |  |  |  |  |  |  |
| Net req'ts |  |  |  | 100 | 80 |  | 60 | 60 |  |
| P.O receipts |  |  |  | 100 | 80 |  | 60 | 60 |  |
| P.O Releases |  |  | 100 | 80 |  | 60 | 60 |  |  |
|  | Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| B | Gross req'ts |  |  |  |  | 50 | 40 | 80 |  |
| On hand |  |  |  |  |  |  |  |  |  |
| Net req'ts |  |  |  |  | 50 | 40 | 80 |  |  |
| P.O receipts |  |  |  |  | 50 | 40 | 80 |  |  |
| P.O Releases |  |  |  | 50 | 40 | 80 |  |  |  |
|  | Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| C | Gross req'ts |  |  | 200 | 160 |  | 120 | 120 |  |
| On hand | 30 | 30 | 30 | 30 | 30 | 30 | 30 | 30 |  |
| Net req'ts |  |  | 170 | 130 |  | 90 | 90 |  |  |
| P.O receipts |  |  | 200 | 160 |  | 120 | 120 |  |  |
| P.O Releases | 200 | 160 |  | 120 | 120 |  |  |  |  |
|  | Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| D | Gross req'ts |  | 120 | 200 | 400 |  | 120 | 120 |  |
| On hand | 100 |  |  |  |  |  |  |  |  |
| Net req'ts |  | 20 | 200 | 400 |  | 120 | 120 |  |  |
| P.O receipts |  | 20 | 200 | 400 |  | 120 | 120 |  |  |
| P.O Releases | 20 | 200 | 400 |  | 120 | 120 |  |  |  |

|  | Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E | Gross req'ts |  |  |  | 50 | 40 | 80 |  |  |
| On hand | 100 | 100 | 100 | 50 | 10 | 0 | 0 | 0 |  |
| Net req'ts |  |  |  |  |  | 70 |  |  |  |
| P.O receipts |  |  |  |  |  | 70 |  |  |  |
| P.O Releases |  |  |  |  | 70 |  |  |  |  |
|  | Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| F | Gross req'ts |  |  |  | 50 | 40 | 80 |  |  |
| On hand | 30 | 30 | 30 | 40 | 0 | 40 | 40 | 40 |  |
| Net req'ts |  |  |  | 20 |  | 80 |  |  |  |
| P.O receipts |  |  |  | 60 |  | 120 |  |  |  |
| P.O Releases |  | 60 |  | 120 |  |  |  |  |  |
|  | Week | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| G | Gross req'ts |  | 60 |  | 120 |  |  |  |  |
| On hand |  |  |  |  |  |  |  |  |  |
| Net req'ts |  | 60 |  | 120 |  |  |  |  |  |
| P.O receipts |  | 60 |  | 120 |  |  |  |  |  |
| P.O Releases | 60 |  | 120 |  |  |  |  |  |  |

Comment

- An auto body shop has five jobs to process through its facilities over the next few weeks. The processing time and due dates for the five jobs are shown in the following table.											    (20 Marks)

| Job | Processing Time (days) | Due Date (days hence) |
| --- | --- | --- |
| A | 7 | 19 |
| B | 8 | 27 |
| C | 12 | 26 |
| D | 5 | 11 |
| E | 9 | 18 |

Knowing that the firm operates seven days per week, assign the jobs according to each of the following priority rules:

- shortest operation time (SOT)

Solution

Sequence using the short Operating time (SOT) is;

D-A-B-E-C

| Job | In Time | Out Time | Due Date | Lateness |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Earliness | Trdness |  |
| D | 0 | 5 | 11 | -6 |  |  |
| A | 5 | 12 | 19 | -7 |  |  |
| B | 12 | 20 | 27 | -7 |  |  |
| E | 20 | 29 | 18 |  |  | 11 |
| C | 29 | 41 | 26 |  |  | 15 |

Total flow time =5+12+20+29+41

=107 days

- earliest due date (EDD)
Solution :

Sequence Using Earliest Due Date (EDD)

Is;

D-E-A-C-B

| Job | In Time | Out Time | Due Date | Lateness |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Earliness | Trdness |  |
| D | 0 | 5 | 11 | -6 |  |  |
| E | 5 | 14 | 18 | -4 |  |  |
| A | 14 | 21 | 19 |  |  | 2 |
| C | 21 | 33 | 26 |  |  | 7 |
| B | 33 | 41 | 27 |  |  | 14 |

Total flow time = 5+14+21+33+41

=114 days

- slack time remaining (STR)

Solution

| Job | Processing Time (P) | Due Date (D) | Due date-Processing time (D-P) Slack Time |
| --- | --- | --- | --- |
| A | 7 | 19 | 12 |
| B | 8 | 27 | 19 |
| C | 12 | 26 | 14 |
| D | 5 | 11 | 6 |
| E | 9 | 18 | 9 |

Lesser time slacR time (D-P), higher the Priority.

Sequence using slacR time remaining is

D-E-A-C-B

| Job | In Time | Out Time | Due Date | Lateness |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Earliness | Trdness |  |
| D | 0 | 5 | 11 | -6 |  |  |
| E | 5 | 14 | 18 | -4 |  |  |
| A | 14 | 21 | 19 |  |  | 2 |
| C | 21 | 33 | 26 |  |  | 7 |
| B | 33 | 33 | 27 |  |  | 14 |

Total Flow Time = 5+14+21+33+41

=114 days

- Which one of the priority rules do you determine to be best in this particular case? Explain?

Solution

Based on the least total flow time, Shortest Operation Time (SOT)  MUST be choosen.

- A water vending machine at the local supermarket is designed to fill 20-litre water bottles. Every 5 hours, a sample of 5 water bottles is taken and the contents of the bottles are carefully measured. While the filling process was in control, the average of the sample means was found to be 19.85 L and the average of the sample range was found to be 0.3 L.									(15 Marks)

- Calculate 3σ control limits for mean and range charts to monitor this process.
Solution

X-BAR = 19.85
R-BAR = 0.3

A2 VALUE CORRESPONDING TO N = 5 = 0.577
D3 & D4 VALUES CORRESPONDING TO N = 5, D3 = 0 & D4 = 2.115

FOR X
UCL = XBAR + (A2 * RBAR) = 19.85 + (0.577 * 0.3) = 20.023
LCL = XBAR - (A2 * RBAR) = 19.85 - (0.577 * 0.3) = 19.677

FOR R
UCL = RBAR * D4 = 0.3 * 2.115 = 0.635
LCL = RBAR * D3 = 0.3 * 0 = 0

- Recently, six additional samples of size 5 have been filled by the vending machine. The individual amounts of water in litres are recorded in the following table. Calculate the mean and the range for each sample.

| # SAMPLE | 1 | 2 | 3 | 4 | 5 | AVERAGE | RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 19.6 | 19.7 | 20.2 | 19.9 | 20.1 | 19.9 | 0.6 |
| 2 | 20.1 | 20 | 19.9 | 20.2 | 19.8 | 20 | 0.4 |
| 3 | 19.7 | 20.2 | 19.8 | 19.6 | 20 | 19.86 | 0.6 |
| 4 | 19.8 | 20.3 | 20.1 | 20.1 | 20.4 | 20.14 | 0.6 |
| 5 | 20 | 20.2 | 20 | 20.2 | 20.3 | 20.14 | 0.3 |
| 6 | 19.7 | 20.1 | 20.1 | 19.9 | 20.2 | 20 | 0.5 |
| AVERAGE |  |  |  |  |  | 20.007 | 0.5 |

AVERAGE X = 20.007

AVERAGE R = 0.5

THE PROCESS APPEARS TO BE UNDER CONTROL.

A2 FOR N = 5 = 0.577

D3 FOR N = 5 = 0, D4 = 2.115

FOR X

UCL X = X + (A2 * R) = 20.007 + (0.577 * 0.5) = 20.296

LCL X = X - (A2 * R) = 20.007 - (0.577 * 0.5) = 19.719

FOR R

UCL R = R * D4 = 0.5 * 2.115 = 1.058

LCL R = R * D3 = 0.5 * 0 = 0

- Draw the mean and the range charts for this process. Plot the results of the six samples on the chart.

- Does the process appear to be in control?

- A bank has found that when their process for completing loan applications is operating in control, the proportion of applications containing errors is 0.03 (i.e. 3%).  To monitor the process, the bank plans to take a sample of 20 applications each day and evaluate the proportion of applications in the sample with errors.

Calculate 3σ control limits for a p-chart to monitor this process.		(5 Marks)

Solution

Given

Sample size (n) =20

Average proportion (p) = Proportion of applications containing Error

=0.03

Now,for  3 σ Limits ,we have  z=3

Control limits Are;

UcLP = P+2 Sqrt(p-(1-p) /n    = 0.03+3Sqrt (0.03(1-0.03)/20

=0.177

LCLp= p-z Sqrt (p(1-p)/n  = 0.03 -3Sqrt 0.03(1-0.03) /20

=0

- Analyze and solve the following project network. The numbers in the top cell of the boxes represent the time in weeks required to perform each activity.	(16 Marks)

- Using the two-step procedure consisting of forward and backward passes, determine the ES/EF and LS/LF of each activity. Also, determine how long it will take to complete the entire project.

A

F

C

B

D

E

2

11

10

4

4

8

Solution

Forward pass:

ES of the starting activity = 0
ES of all others = Max. (EF of their predecessors)
EF = ES + Duration

| Activity | Duration | ES | EF |
| --- | --- | --- | --- |
| A | 2 | 0 | 2+0 = 2 |
| B | 11 | 2 | 11+2 = 13 |
| C | 10 | 2 | 10+2 = 12 |
| D | 4 | Max(12,13) = 13 | 4+13 = 17 |
| E | 4 | 12 | 4+12 = 16 |
| F | 8 | Max(16,17) = 17 | 8 + 17 = 25 |

Backward pass:

LF of the ending activity = Max of all EFs
LF of all others = Min.(LS of their successors)
LS = LF - Duration

| Activity | Duration | LS | LF |
| --- | --- | --- | --- |
| A | 2 | 2 - 2 = 0 | Min(2,3) = 2 |
| B | 11 | 13 - 11 = 2 | 13 |
| C | 10 | 13 - 10 = 3 | Min(13,13) = 13 |
| D | 4 | 17 - 4 = 13 | 17 |
| E | 4 | 17 - 4 = 13 | 17 |
| F | 8 | 25 - 8 = 17 | 25 |

Total time required to complete the project = LF or EF of activity-F = 25 weeks

- Which activities are on the critical path? Calculate slack times for any activity that is not on the critical path.

Slack = LF - EF for all activities

| Activity | Slack |
| --- | --- |
| A | 2 - 2 = 0 |
| B | 13 - 13 = 0 |
| C | 13 - 12 = 1 |
| D | 17 - 17 = 0 |
| E | 17 - 16 = 1 |
| F | - - 15 = 0 |

The critical path is constituted by activities with zero slack. So, the critical path is A-B-D-F

c).If duration of activity E increases by one week, will the completion of the project be delayed? Explain.

Solution

Activity-E has a slack of 1 week. So, a delay of 1 week will not delay the project.

If activity E takes six weeks to complete, will the completion of the project be delayed? If so, what is the new project duration?

Solution

If E takes 6 weeks, the delay will be 2 weeks. Since the slack is 1 week, the delay in the project will be 2 - 1 = 1 week. So, the new project duration will be 26 weeks.