6 problems

12.05  and 12.39 is not in the book. Therefore I do not have the prolems

Problem 12.05  page 507-508

Demand D = 19,500 units / year

Ordering cost S = $25 / order

Holding cost H = $4 / unit / year

- EOQ = 2 19,500254 = 493.71 = 494 units

- Annual holding costs = (Q / 2)H = (494 / 2)4 = $988

- Annual ordering costs = (D / Q)S = (19500 / 494)25 = $987

Problem 12.09  page 508

(a)

This problem reverses the unknown of a standard EOQ problem to solve for S.

60 = 2 x 240 x S.4 x 10 ; or 60 = 480S4 , or 60 = 120S, so solving for S results in S = 3,60012 = $30.

(b)

If S were $30, then the EOQ would be 60. If the true ordering cost turns out to be much greater than $30, then the firm´s order policy is ordering too little at a time.

Problem 12.11  page 508

Yearly demand, D = 15,000 units

Annual holding cost, H = $25 per unit

Cost order from the supplier, S = $75.

The lead time, L = 2 day

a)

𝐸𝑂𝑄 = √2𝐷𝑆/𝐻

= √(2 ∗ 15,000 ∗ 75)/25

= 300 units

The economic order quantity, EOQ = 300 units

b)

The annual holding cost = (EOQ/2) * H = (300/2) * 25 = $3,750

c)

The annual ordering costs = (D/EOQ) * S = (15,000/300) * 75 = $3,750

d)

Lead time, L = 2 days

ROP = d*L

= (15,000/300) * 2 = 100 units

Problem 12.13  page 508

a)

1000/300=33.333

5*33.333=166.67

Therefore, the reorder point for the notebook binders that she stocks are 167 binders.

b)

The number is important to make sure they maintain enough inventories of the binders to cover the 5 working daytime frame that it takes to receive the order from the supplier.  167 binders will cover the 5 working daytime frame.

Problem 12.15  page 508

a)

EOQ= sqrt((2DS)/H)

=sqrt( (2*2,500*18.75)/1.5)

=Sqrt (93,750/1.5)

=sqrt (62,500)

=250 units

b)

Average inventory = EOQ/2

=250/2

=125 units

Average inventory is 125 units

Inventory holding cost = (EOQ/2)H

=(250/2)*1.5

=25*1.5

=$187.50

Inventory holding cost is $187.50

c)

Number of orders per year = D/EOQ

=2,500/250

=10

Number of orders =10

Annual cost of ordering = 10* $18.75

=$187.50

Annual cost of ordering is $187.50

d)

Cost of managing inventory = Inventory cost + annual cost of ordering

=$187.50 + $187.50

=$375 per year

e)

The time between orders=num working days/num of orders

=250/10

=25 days

f)

ROP= Demand per day * lead time

=(2,500/250)

=10

=10 Units

Problem 12.39  page 511

a)

Safety stock for 90% level=7*[60 at 0.90]=7*1.28=8.96=9

b)

Reorder point=60+9=69 bandages.

Problem 14.09  page 581

| Item S Period | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gross reuirements | 0 | 0 | 0 | 0 | 0 | 0 | 100 |
| Scheduled receipts | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beginning inventory | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Ending inventory | 20 | 20 | 20 | 20 | 20 | 20 | 0 |
| Net reuirement | 0 | 0 | 0 | 0 | 0 | 0 | 80 |
| Planned order receipts | 0 | 0 | 0 | 0 | 0 | 0 | 80 |
| Planned order releases | 0 | 0 | 0 | 0 | 80 | 0 | 0 |
| Item U Period | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| Gross reuirements | 0 | 0 | 0 | 0 | 160 | 20 | 0 |
| Scheduled receipts | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Beginning inventory | 40 | 40 | 40 | 40 | 40 | 0 | 0 |
| Ending inventory | 40 | 40 | 40 | 40 | 0 | 0 | 0 |
| Net reuirement | 0 | 0 | 0 | 0 | 120 | 20 | 0 |
| Planned order receipts | 0 | 0 | 0 | 0 | 120 | 20 | 0 |
| Planned order releases | 0 | 0 | 120 | 20 | 0 | 0 | 0 |

Problem 14.11  page 581

a) Develop a gross requirements plan for Item C.

To develop a Gross Requirements Plan for item C, we note from the diagram that one of item C is used in Assembly A and another is used in Assembly H. Note that existing inventories aren’t taken into account in a gross requirements plan. Our plan should thus show requirements for all three components:

| Item |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Gross Req |  |  |  |  |  |  |  | 100 |  | 50 |  | 150 |
|  | Order Rel |  |  |  |  |  |  | 100 |  | 50 |  | 150 |  |
| H | Gross Req |  |  |  |  |  |  |  |  | 100 |  | 50 |  |
|  | Order Rel |  |  |  |  |  |  |  | 100 |  | 50 |  |  |
| C | Gross Req |  |  |  |  |  |  | 100 | 100 | 50 | 50 | 150 |  |
|  | Order Rel |  |  |  |  | 100 | 100 | 50 | 50 | 150 |  |  |  |

b) A net requirements plan for Item C.

Our net requirements plan does taken into account existing inventories and provides us with some more detailed quantities. Once again, it should show items A, H, and C:

| Item |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Gross Req |  |  |  |  |  |  |  | 100 |  | 50 |  | 150 |
|  | On Hand |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Net Req |  |  |  |  |  |  |  | 100 |  | 50 |  | 150 |
|  | Planned Receipt |  |  |  |  |  |  |  | 100 |  | 50 |  | 150 |
|  | Planned Release |  |  |  |  |  |  | 100 |  | 50 |  | 150 |  |
| H | Gross Req |  |  |  |  |  |  |  |  | 100 |  | 50 |  |
|  | On Hand |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Net Req |  |  |  |  |  |  |  |  | 100 |  | 50 |  |
|  | Planned Receipt |  |  |  |  |  |  |  |  | 100 |  | 50 |  |
|  | Planned Release |  |  |  |  |  |  |  | 100 |  | 50 |  |  |
| C | Gross Req |  |  |  |  |  |  | 100 | 100 | 50 | 50 | 150 |  |
|  | On Hand | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 0 | 0 | 0 | 0 | 0 |
|  | Net Req |  |  |  |  |  |  | 50 | 100 | 50 | 50 | 150 |  |
|  | Planned Receipt |  |  |  |  |  |  | 50 | 100 | 50 | 50 | 150 |  |
|  | Planned Release |  |  |  |  | 50 | 100 | 50 | 50 | 150 |  |  |  |

Problem 14.13  page 581

a)	Construct a product structure.

The product structure diagram is:

20” Fan

Switch

Motor

Blades (5)

Hub

Handle

Knob

Supports (2)

Frame

Electrical

Housing

Grills (2)

Fan Assembly

b) The time-phased product structure is:

|  |  | Frame |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Support | Housing |  |  |
|  |  |  | Handle |  |  |  |
|  |  |  | Grills |  | Fan |  |
|  | Hub |  |  |  |  |  |
|  |  | Fan Assembly |  |  |  |  |
| Blades |  |  |  |  |  |  |
|  |  |  | Motor |  |  |  |
|  |  |  | Switch | Elec |  |  |
|  |  |  | Knobs |  |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

c) The net material requirements plan is:

| Lot Size | Lead Time | On Hand | Item ID |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 100 | 20” Fan | Gross Requirements |  |  |  |  |  |  | 1000 |
|  |  |  |  | Projected On Hand |  |  |  |  |  |  | 100 |
|  |  |  |  | Net Requirements |  |  |  |  |  |  | 900 |
|  |  |  |  | Planned Order Receipts |  |  |  |  |  |  | 900 |
|  |  |  |  | Planned Order Releases |  |  |  |  |  | 900 |  |
|  | 1 | 100 | Housing | Gross Requirements |  |  |  |  |  | 900 |  |
|  |  |  |  | Projected On Hand |  |  |  |  |  | 100 |  |
|  |  |  |  | Net Requirements |  |  |  |  |  | 800 |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  |  | 800 |  |
|  |  |  |  | Planned Order Releases |  |  |  |  | 800 |  |  |
|  | 2 | 200 | Grills | Gross Requirements |  |  |  |  |  | 1800 |  |
|  |  |  |  | Projected On Hand |  |  |  |  |  | 200 |  |
|  |  |  |  | Net Requirements |  |  |  |  |  | 1600 |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  |  | 1600 |  |
|  |  |  |  | Planned Order Releases |  |  |  | 1600 |  |  |  |
|  | 3 | 150 | Fan Assembly | Gross Requirements |  |  |  |  |  | 900 |  |
|  |  |  |  | Projected On Hand |  |  |  |  |  | 150 |  |
|  |  |  |  | Net Requirements |  |  |  |  |  | 750 |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  |  | 750 |  |
|  |  |  |  | Planned Order Releases |  |  | 750 |  |  |  |  |
|  | 1 |  | Electrical Unit | Gross Requirements |  |  |  |  |  | 900 |  |
|  |  |  |  | Projected On Hand |  |  |  |  |  |  |  |
|  |  |  |  | Net Requirements |  |  |  |  |  | 900 |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  |  | 900 |  |
|  |  |  |  | Planned Order Releases |  |  |  |  | 900 |  |  |
|  | 2 |  | Frame | Gross Requirements |  |  |  |  | 800 |  |  |
|  |  |  |  | Projected On Hand |  |  |  |  |  |  |  |
|  |  |  |  | Net Requirements |  |  |  |  | 800 |  |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  | 800 |  |  |
|  |  |  |  | Planned Order Releases |  |  | 800 |  |  |  |  |

| Lot Size | Lead Time | On Hand | Item ID |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 50 | Supports | Gross Requirements |  |  |  |  | 1600 |  |  |
|  |  |  |  | Projected On Hand |  |  |  |  | 50 |  |  |
|  |  |  |  | Net Requirements |  |  |  |  | 1550 |  |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  | 1550 |  |  |
|  |  |  |  | Planned Order Releases |  |  |  | 1550 |  |  |  |
|  | 1 | 400 | Handle | Gross Requirements |  |  |  |  | 800 |  |  |
|  |  |  |  | Projected On Hand |  |  |  |  | 400 |  |  |
|  |  |  |  | Net Requirements |  |  |  |  | 400 |  |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  | 400 |  |  |
|  |  |  |  | Planned Order Releases |  |  |  | 400 |  |  |  |
|  | 1 |  | Hub | Gross Requirements |  |  | 750 |  |  |  |  |
|  |  |  |  | Projected On Hand |  |  |  |  |  |  |  |
|  |  |  |  | Net Requirements |  |  | 750 |  |  |  |  |
|  |  |  |  | Planned Order Receipts |  |  | 750 |  |  |  |  |
|  |  |  |  | Planned Order Releases |  | 750 |  |  |  |  |  |
| 100 | 2 |  | Blades | Gross Requirements |  |  | 3750 |  |  |  |  |
|  |  |  |  | Projected On Hand |  |  |  | 50 | 50 | 50 | 50 |
|  |  |  |  | Net Requirements |  |  | 3750 |  |  |  |  |
|  |  |  |  | Planned Order Receipts |  |  | 3800 |  |  |  |  |
|  |  |  |  | Planned Order Releases | 3800 |  |  |  |  |  |  |
|  | 1 |  | Motor | Gross Requirements |  |  |  |  | 900 |  |  |
|  |  |  |  | Projected On Hand |  |  |  |  |  |  |  |
|  |  |  |  | Net Requirements |  |  |  |  | 900 |  |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  | 900 |  |  |
|  |  |  |  | Planned Order Releases |  |  |  | 900 |  |  |  |
| 12 | 1 | 20 | Switch | Gross Requirements |  |  |  |  | 900 |  |  |
|  |  |  |  | Projected On Hand |  |  |  |  | 20 | 8 |  |
|  |  |  |  | Net Requirements |  |  |  |  | 880 |  |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  | 888 |  |  |
|  |  |  |  | Planned Order Releases |  |  |  | 888 |  |  |  |

| Lot Size | Lead Time | On Hand | Item ID |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 1 |  | Knob | Gross Requirements |  |  |  |  | 900 |  |  |
|  |  |  |  | Scheduled Receipt |  | 200 |  |  |  |  |  |
|  |  |  |  | Projected On Hand |  |  | 200 | 200 | 200 |  |  |
|  |  |  |  | Net Requirements |  |  |  |  | 700 |  |  |
|  |  |  |  | Planned Order Receipts |  |  |  |  | 700 |  |  |
|  |  |  |  | Planned Order Releases |  |  |  | 700 |  |  |  |