MPT 651 – MUNICIPAL ENGINEERING SYSTEMS
Final Exam – April 21st , 2021  8:00 am – 8:00 pm
Name:







· Total marks = 100
· You may use the back of the exam pages as additional work space.
· Total time = 2 hours

1) Name and briefly describe the two (2) types of pumping stations (wells) when dealing with forcemains and describe the difference between the two. (3)
2) What are the THREE main goals of Stormwater Management? (3)
3) Identify each of the following equations (name them): (4)
[image: image1.wmf]25

1

300

.

.

´

÷

ø

ö

ç

è

æ

+

=

D

I

t


[image: image2.png]


Q = CIA / 360  
[image: image3.wmf]25

1

300

.

.

´

÷

ø

ö

ç

è

æ

+

=

D

I

t

[image: image4.png]



[image: image5.png]10.675-L- Q"%
ST s s



Q = AR2/3 S1/2 n-1 




            of 10 pts

4) When designing a road, list Traffic Factors that influence pavement design: (5)
5) List factors that influence the type and severity of erosion. (4)





6) List as many E&SC BMP’s as you can. (8)





7) List the three (3) types of basic methods for protecting workers against cave-ins. (3)






            of 20 pts

8) Calculate the height of a trench box for a 2100mm storm sewer. (10)
· Maintain 600mm clearance on either side of the pipe
· Maintain 600mm bench at top of the shoring system.

· Maintain 300mm clearance from top of soil to top of trench box.

· Label all dimensions in mm.


9) A town is 3.5 km from its water source and 12m above it. The watermain between the town and the water source is 375 mm in diameter with a roughness coefficient of 110. The starting pressure at the water source is 800 kPa and the residual pressure at town is 300 kPA. Water is also stored in a water tower that is 2.5 km beyond the town with an elevation of 70m. The watermain between the town and the water tower is 250 mm in diameter (C = 100). Specific weight is 9.8 as always. Velocity is negligible. (10)
· Determine QT at the town.  (PLEASE SHOW SOME CALCULATIONS)
· NOTE:  I HIGHLY RECOMMEND YOU HAND DRAW / SKETCH A DIAGRAM TO MAKE SURE YOU SET UP THE PROBLEM CORRECTLY
10) Please complete the design spreadsheet for the sanitary sewer layout plan provided (next 2 pages). You’ll need to size ALL (five in total) legs of sanitary sewer ensuring the following: (25)
a) Flow within the pipe cannot exceed 80% of the capacity of the pipe;

b) Assume n = 0.013;

c) Assume 450 L/cap/day
d) You must show at least one sewer leg’s worth of detailed calculations (by hand) in order to get full marks (place calculations below).

(ADDITIONAL WORKSPACE AS REQ’D)
11) Please complete the design spreadsheet for the storm sewer layout and drainage plan provided  (25)
a) Flow within the pipe cannot exceed 80% of the capacity of the pipe;

b) Assume n = 0.013;

c) You must show at least one sewer leg’s worth of detailed calculations in order to get full marks (place calculations below).  BE SURE TO CALCULATE THE ACTUAL VELOCITY BY HAND (SHOW WORK) FOR THE FIRST LEG (this is noted on spreadsheet)  
(ADDITIONAL WORKSPACE AS REQ’D)































� EMBED Equation.3  ���






























































_1489832993.unknown

