Virginia States University

Department of Engineering and Computer Science

ENGR 101-02 – Introduction to Engineering (Fall 2020)

Date:

Test #: 1 (Chapters 4 and 5) Section 2

On my honor, I have neither given nor received any unauthorized aid on this test, and I pledge that I am in compliance with the VSU Honor System.

Print Name	                                                        Signature

Section 2: Read each question carefully and show your work to get full credit.

- Determine the magnitude and direction of the resultant force by adding the rectangular component of each force (that is by resolving the force their x and y components). (10 points)

FR = 29.67N

- Consider the three forces, F1 = 5 i + 2 j kN, F2 = −3 i − 8 j kN and F3 = −4 i + 7 j kN. Find the resultant force, its magnitude, and direction with respect to the positive x-axis. (5 points)

FR = F1 + F2 + F3

FR = (5 i + 2 j + (−3 i − 8 j) + (−4 i + 7 j)) kN

FR = (5 i −3 i −4 i) +(2 j − 8 j + 7 j)

FR = -2i  + j

FRx = -2i

FRy = j

FRx = -2i  kN

- A 30-cm diameter solid steel sphere hangs from cables as shown below. Find the tension in cables AB and AC. For the density of steel, use ρ = 7270 kg/m3. Use the “General Analysis Procedure” to present your solution. The general analysis procedure is the 7 step method engineers use to solve problems in a logical and systematic matter. (10 points)

- Two cables, AB and AC has cross sectional diameters of 0.004 m and 0.006 m and tensions 2338 N and 3052 N, respectively. What is the normal stress in each cable? (5 points)

For Cable AB, normal stress =  2338π*0.0022 = 186 .05 * 106 N/m2

For Cable AC, normal stress =  3052π*0.0032 = 107.94* 106  N/m2

- A typical stress-strain diagram is shown below. Use the following words or phrase to identify the labelled regions: (5 points)

Yield stress                                       Elastic limit                        fracture

Compressive stress                           proportional limit               Ultimate stress

Shear stress                                       tensile strain                       proportional limit

O-A =   proportional limit

A= Elastic limit

B=   Yield stress

C =  Shear stress

D= Ultimate stress

E=    fracture

- What is the normal deformation in the cables from the previous question (question 18), if the length of each cable is 0.7 m and the modulus of elasticity, E is 200 GPa? ( Hint*1 Nm2=1 Pa   and  1 kPa=1000 Pa) (5 points)

For Cable AB, normal stress =  2338π*0.0022 = 186 .05 * 106 N/m2

For Cable AC, normal stress =  3052π*0.0032 = 107.94* 106  N/m2

δ=PLAE       Where E is the modulus of elasticity

A is the cross-sectional area

δ : normal deformation and L: original length

P is internal force

For cable AB

δ=0.7*186 .05 * 106 200*109  = 6.511* 10-4m

For cable AC

δ=0.7*107.94* 106 200*109  = 3.78* 10-4m

- A circuit element absorbs 6 W of power due to the passage of a steady current of 250 mA. What is the voltage across the element?  (5 points)

P = VI

Where V= Voltage

I = current

P = Power

250mA= 0.25A

V = PI = 6W0.25A = 24 V

- Calculate the current I3 in the following circuit where I1, I2, I4 and I5 are 15 A, 12A, 3A and 7A respectively. (5 points)

Sum of total current = Zero

Current into the node = Current out of the node

I1 +  I4 + I2 = +  I3+ I5

15 + 3 + 12 =  + 7  + I3

I3= 30- 7 = 23 A

I3= 23A

- Four resistors are connected in parallel across a 200-mA independent current source as shown below. What is the voltage across the resistors and the current in each resistor? (10 points)

1RT=1R1+1R2+1R3+…= 1RT=180+11000+1100 +1600= 1516000

Rt=  6000151=39.735Ω

V=IR=0.2A*39.735=7.95 V

Voltage across the resistors = 7.95V

Current through 80Ω = 7.9580 = 0.0994 A

Current through 1000Ω = 7.951000 = 0.00795A

Current through 100Ω = 7.95100 = 0.0795A

Current through 600Ω = 7.95600 = 0.01325A

- For the DC circuit shown in the accompanying figure, find the voltage across each resistor and the current through each resistor. (15 points)

Useful Equations

- Sine Rule

The Sine Rule is used in the following cases:

CASE 1: Given two angles and one side (AAS or ASA)

CASE 2: Given two sides and a non-included angle (SSA)

The Sine Rule states that the sides of a triangle are proportional to the sines of the opposite angles. In symbols,

- Cosine Rule (The Law of Cosine)

## The Cosine Rule is used in the following cases:

- Given two sides and an included angle (SAS)

2.  Given three sides (SSS)

The Cosine Rule states that the square of the length of any side of a triangle equals the sum of the squares of the length of the other sides minus twice their product multiplied by the cosine of their included angle. In symbols:

- Vector Components:

  - Ax=A cosα

Ay=A sin α

Bx=B cosβ

By=B sin β

  - Writing the vector components Ax and Ay in terms of a scalar and a unit vector as: Ax=Axi  and Ay=Ayj. Thus, vectors A is expressed as:

A=Axi+Ayj

And vector B is expressed as:

B=Bxi+Byj

The resultant of A and B is:

R=A+B=Ax+Bxi+(Ay+Byj

The rectangular components of the resultant vector R are given as:

Rx=Ax+Bx

and

Ry=Ay+By

Hence we can write the result of the vectors A and B as:

R=Rxi+Ryj

The magnitude of the result vector is:

R=Rx2+Ry2

The direction of the resultant vector is the value of the angle θ:

Tan θ= RyRx

- F=0   (the equation for equilibrium

- σ=PA        Where σ is normal stress, P is internal force and A is the cross-sectional area

- ε= δL        Where ε: normal strain, δ : normal deformation and L: original length

- δ=PLAE       Where E is the modulus of elasticity; the other variables are already defined.

- Area of circle= πr2

- I= qt

- 1e=1.602×10-19 C

- V=Wq

- V=IR

- P=IV=I2R=V2R

- Rs=R1+R2+R3+………..

- 1Rp=1R1+1R2+1R3+………..

- Iin=0=I2+I2+I3+I4+I5

- Iin=0

- V=0