Paper Recycling

Name

Affiliation

Date

Paper Recycling

Paper recycling is one of the popular industries in the united states because of the growing investment in saving the world. Recycling makes up a massive segment of the industry through its range of activities linked with the recovery and processing of scrap paper. According to the environmental protection agency, recycling paperboards and papers increased from about 5 million tons to 45 million tons by the end of 2017.almost all kinds of writing are recyclable, not including pizza boxes, paper towels, and carbon paper, and kraft envelopes.

The paper recycling process involves several processes that begin with collection, conveyance, and sorting. Waste papers are collected from different bins and deposited in a large recycling container. After transportation, sorting the material into diverse categories such as computer papers, magazine papers, and newspapers follows. The processing of sorted forms makes the slurry. The pulping process involves chemicals and water. During this process, the machines chop the documents before the addition of chemicals and water. The mixture is then heated to break down the pulp into paper fibers. Pulp cleaning and screening aim at removing contamination from the pulp. The slurry is forced through holes of different shapes and sizes to remove bits of plastics and glob of glue. If the pulp contains heavy containment such as staples, the slurry may be spun around in huge cylinders. The centrifugal force inside the cylinders throws away the severe containment (Rahmasari, 2020).

The next stage is di-inking. in this stage, the separation of sticky materials and removing ink from fibers occurs. De-inking involves a combination of chemicals and mechanical actions such as shredding. Water removes small and light ink materials, while flotation removes the larger particles. In the refining stage, the breaking down of pulp into paper fiber swell occurs. The separation of fiber happens by beating the pulp to facilitate paper production. In case coloring is required, color stripping chemicals are added to the threads to remove the dyes present in the pulp. Finally, the pulp is bleached with hydrogen peroxide, chlorine dioxide, and oxygen to make them whiter and brighter. In the final phase of paper recycling, the cleaned pulp is blended with wood fibers to offer new paper additional strength and smoothness. after mixing the pulp with hot water and chemicals, the mixture is fed into the paper-making machine. The squeezing water from the pulp produces new sheets of paper (Bajpai, 2013).

Chemical pulping removes lignin from lignocellulose. The product is used for paper production, while the chemical cellulose helps manufacture carboxymethyl cellulose. The chemical pulping process hydrolyzes the hemicellulose. The pulping process requires the crushing of lignin into smaller fragments that can be washed away from the fiber. Besides, advanced chemical pulping uses an organic solvent. Rafting pulping removes lignin through an alkaline process involving the action of sodium sulfide and sodium hydroxide. The digestor operates at almost 175 degrees Celsius for nearly 4 hours. The sodium sulfide inhibits the condensation reactions while promoting cleavage.  The apparent density, bursting strength, and tensile strength of recycled papers reduce immediately after the second recycle. The WRV and carboxyl content of the pulp also reduces. Chemical treatment on pulp massively reduces the physical properties of the recycled papers (Lehto & Alén, 2015).

References

Bajpai, P. (2013). Recycling and Deinking of recovered paper. Elsevier Science.

Lehto, J. T., & Alén, R. J. (2015). Chemical Pretreatments of wood chips prior to alkaline pulping - A review of Pretreatment alternatives, chemical aspects of the resulting liquors, and pulping outcomes. BioResources, 10(4). https://doi.org/10.15376/biores.10.4.lehto

Rahmasari, F. (2020). The effect of phase on paper thickness optimization in the paper recycling process using recycling machine. IJIS Edu : Indonesian Journal of Integrated Science Education, 2(1), 19. https://doi.org/10.29300/ijisedu.v2i1.2564