Physics Homework 5 (Answers)

Student’s Name

Date

Physics Homework 5 (Answers)

Q1) Total torque on left side = TA +TB

= FA x rA + FB x rB

=100 ft.72 kips+40 ft.45.kips

= 7200 +1800 ft kips

= 9000 ft kips

Right side needs to equal left

- T1 = 9000 ft kips

- Fc x rc = 9000 ft kips

- 30 ft.Fc = 9000 ft kips

- Fc = 300 kips

Q2. Conversion: 1 pound (lb , lbs) = 4.45 newtons earth (N)

Which means 3500 lbs. = 15,575 N

AC cos (45) - BC cos (30) = 0

AC sin (45) – BC sin (30) – 15,575 N = 0

Using these equations, we can solve for the resultant force of two cables.

AC =  BCcos (30)cos (45)

BCcos (30)cos (45) sin (45) – BC sin (30) – 15,575 N = 0

Therefore, the resultant forces of two cables are:

AC = 11,013.31 N or 2,475 lbs.

BC = 7787.60 N or 1,750 lbs.

Q3. Conversion: 1 kip is equal to 4448.2216 newtons earth (N)

Which means 160 kips = 711,715.5 N

AC cos(45) - BC cos(45) = 0

AC sin(45) – BC sin(45) – 711,715.5 N = 0

Using these equations, we can solve for the resultant force of two cables.

AC =  BCcos (45)cos (45)

=  BCcos (45)cos (45)   sin(45) – BC sin(45) – 711,715.5N = 0

Therefore, the resultant forces of two cables are:

AC = BC = 503,264.2 N or 113 kips.