Preliminary Hazard Analysis

Student’s Name

Institutional Affiliation

IENG 461 – System Safety 1

October 17, 2017

Preliminary Hazard Analysis

Part 1

- Conveyor Belt

| Hazard | Cause | Consequence | Risk Reduction | Assigned to: |
| --- | --- | --- | --- | --- |
| Worker’s hand snagged at a nip point (Mechanical) | Reaching into an in-going nip point to remove debris | Major Injury | Administrative Control Prohibit cleaning when the conveyor belt is operating Engineering Controls Building barriers around nip points | Team 1 |
| A worker’s hair or cloth pulled by conveyor belt (mechanical) | Worker too close to the conveyor belt; worker maintaining belt when still operating | Injury or Death | Administrative Control Prohibit maintenance when the conveyer belt is operating Engineering Controls Install emergency switches at strategic locations | Team 2 |
| Electrocution (Electrical) | Individual beneath an overhead conveyor belt whose drive train is exposed. | Death | Engineering Controls Build barriers around train drive Administrative Control Prohibit working under train drives Personal Protective Equipment Wearing helmets | Team 3 |

- Coffee Maker

| Hazard | Cause | Consequence | Risk Reduction | Assigned to: |
| --- | --- | --- | --- | --- |
| Hot coffee spills on an individual causing burns (thermal) | Incompetent handling, leaks | Minor Injury | Administrative Controls Providing a manual for proper handling Substitution Replacing a faulty coffee maker with a less hazardous one | Team 1 |
| Electric shock (electrical) | Deterioration of wire insulation | Minor Injury | Engineering Controls Repairing conducting wires Administrative Control Prohibiting the use of coffee machines outside designated areas | Team 2 |
| Electric Fires (electrical) | Malfunctioning thermostat | Serious Burns, Fatality | Substitution Replacing the faulty coffee maker with a new one Elimination Getting rid of the hazardous coffee maker | Team 3 |

- Iron Box

| Hazard | Cause | Consequence | Risk Reduction | Assigned to: |
| --- | --- | --- | --- | --- |
| House Catches fire (Thermal) | User leaves iron box switched on. | Injury, Death | Administrative Control Limit the use of iron boxes to designated areas only Elimination Eliminating the iron box and using laundry services | Team 1 |
| User suffers a burn (Thermal) | User accidentally touches the hot surface | Major Injury | Administrative Control Assigning the operation of the iron box to experienced individuals only Elimination Using laundry services | Team 2 |
| User electrocuted (electrical) | Deteriorated insulation of wire | Death | Substitution Replacing a faulty iron box with a new one Engineering control Repair of wires with deteriorated insulation | Team 3 |

- Overhead Crane

| Hazard | Cause | Consequence | Risk Reduction | Assigned to: |
| --- | --- | --- | --- | --- |
| Electrocution (electrical) | Inadvertent contact of a metal part with any high voltage power source | Death | Substitution Replacement of overhead crane with jib crane or gantry crane (Bolduc, 2017) Engineering Control Proper insulation of susceptible parts of the crane | Team 1 |
| Overloading (mechanical) | Operator fails to observe weight limit | Injury from structural failure | Substitution Replacement of overhead crane with jib crane or gantry crane (Bolduc, 2017) Administrative Control Formulating stringent rules against overloading | Team 2 |
| Bystanders injured by falling materials (mechanical) | Slipping, operator incompetency, mechanical failure, visual impairment | Serious injury or Fatality | Engineering Controls Repairing worn out parts to ensure a firm grip Administrative Controls Allowing operation of overhead cranes only during low traffic hours | Team 3 |

Part 2

For my class project, I intend to use conveyor belts. The reason behind this choice is the fact that conveyor belts are widely used in almost all industries to move commodities from one point to another. Their wide application can be attributed to the flexibility of their design, their relatively low energy requirement and their capability to transport goods in bulk. They transport these commodities around curves, horizontally, vertically or at an angle. That being said, they pose special hazards associated with mechanical motion, electrical power and the commodities conveyed. As such, the project would present a tremendous opportunity towards the development of new knowledge to enhance their safety.

Reference

Bolduc, J. (2017). Overhead Crane Alternatives | Crane-Tec, Inc.. Crane-tec.com. Retrieved 17 October 2017, from http://www.crane-tec.com/blog/uncategorized/overhead-crane-alternatives/