Name

Professor

Course

School

Rock Mechanics- Assignment 3

Question One

a)

| Parameter | Value | Rating |
| --- | --- | --- |
| Compressive strength | 160 MPa | 12 |
| RQD (%) | 74 | 13 |
| Joint Spacing | 0.24 m | 10 |
| Condition of joints | Steep and rough | 30 |
| Groundwater | u/σ | 7 |
| Rating adjustment | Tunnel driven against dip, dip = 500, fair | -5 |
|  | RMR | 67 |

Detailed classification for the condition of joints;

| Parameter | Value | Rating |
| --- | --- | --- |
| Persistence | Less than 1 mm | 5 |
| Aperture (mm) | None | 6 |
| Roughness | Steep and rough | 6 |
| Infilling | None | 6 |
| Weathering | Unweathered with occasional strains | 6 |
|  | Rating | 29 |

b)

| Parameter | Value | Rating |
| --- | --- | --- |
| Compressive strength | 160 MPa | 12 |
| RQD (%) | 74 | 13 |
| Joint Spacing | 0.24 m | 10 |
| Condition of joints | Steep and rough | 30 |
| Groundwater | u/σ | 7 |
| Rating adjustment | Tunnel driven parallel dip, dip = 500, very unfavorable | -12 |
|  | RMR | 60 |

c)

The minor principal stress is vertical;

σv = σ3 = γ*z = 27*150 = 4050 kN/m2 = 4.05 MPa

The major principal stress is horizontal;

σh = σ1 = 2σ3 = 2*4.05 = 8.1 MPa

With an RMR of 60, the rock mass was classified as Class III, fair rock. The stand-up time of the tunnel, spanning 15 m is about 100 to 1000 hours. The support measure to include is a combination of rock bolts and shotcrete.

Question Two

a) RMR

| Parameter | Value | Rating |
| --- | --- | --- |
| UCS | 255 | 15 |
| RQD (%) | Good (about 75 - 90), take 75% | 17 |
| Joint space | 0.17 m | 8 |
| Condition of joints | Slightly altered with a sand coating | 20 |
| Groundwater | Moist | 10 |
| Rating adjustment | Unfavorable | -10 |
|  | RMR | 60 |

b) Q- index

| Parameter | Value | Formula | Value |
| --- | --- | --- | --- |
| RQD (%) | 85% | RQD | 85 |
| Joint Set Number | Three joint sets, F | Jn | 9 |
| Joint Roughness Number | Rough, planar, E | Jr | 1.5 |
| Joint Alteration Number | Slightly altered with sand strains, C | Ja | 2 |
| Joint Water Reduction Factor | Moist | Jw | 1 |
| Stress Reduction Factor | Competent rock, σc/σ1= 30 | SRF | 1 |
|  | Q |  | 7.08 |

The minor principal stress;

σv = σ3 = γ*z = 25*200 = 5000 kN/m2 = 5 MPa

The major principal stress;

σh = σ1 = 2σ3 = 1.7*5 = 8.5 MPa

σc/σ1 = 255/8.5 = 30

Q= (RQD/Jn)(Jr/Ja)(Jw/SRF) = (85/9)(1.5/2)(1/1) = 7.08

With a Q of 7.08, the rock mass is classified as Class C, fair rock. The correct support measures in the category four reinforcement is a combination of rock bolts and a shotcrete.

c) GSI

| Parameter | Value |
| --- | --- |
| Rock Mass Structure | Three joint sets, between massive and blocky |
| Joint Surface Conditions | Slightly altered, between very good and good |
| GSI | 75±5 |

Question Three

Case A- Plane stress

Out-of-plane normal stress σz is equal to zero.

Given;

| E (MPa) | v | σx (MPa) | σy (MPa) | σz (MPa) |
| --- | --- | --- | --- | --- |
| 10000 | 0.25 | 5 | 50 | 5 |

From the formulas given in the lecture notes, the following results were found for the two cases involved;

|  | Case A | Case B |
| --- | --- | --- |
| ɛx | -0.000875 | -0.000875 |
| ɛy | 0.004750 | 0.00475 |
| ɛz | -0.000875 | -0.000875 |
| γxy | 0.00000005 | 0.000005 |

Further calculations;

|  | Case A | Case B |
| --- | --- | --- |
| Areas (SM) | 0.001 | 0.1 |
| τxy | 5000 | 50 |
| τyz | 50000 | 500 |
| τzx | 5000 | 50 |
| G | 4000 |  |
| σz (MPa) | 0 | 13.75 |
| ɛvol | 0.0028 | 0.0034 |

Question Four

a)

Figure 1. Axial versus confining stresses.

σci = 136.82 MPa

tan2 (45 + ϕ/2) = 8.2773 hence, ϕ = 88.620

σci = 2c tan (45 + ϕ/2) = 2c tan (45 + 88.62/2) = 136.82

c = 0.82 MPa

b)

Figure 2. Square deviator versus confining stresses.

Figure 3. Major versus minor stresses.

(σ1’- σ3’)2 = mi σci σ3’ + σci2

σci2 = 16400 hence, σci = 128.06 MPa

mi σci = 2780.8 hence, mi = 21.71

The theoretical Hoek-Brown envelope is limited to:

σ1 = σ3 + σci {(mi* σ3/ σci + s)0.5

s is 1 for intact rock, hence the equation results in;

σ1 = σ3 + 128.06 {(21.71* σ3/128.06) + 1}0.5

c)

σ3 = 8 MPa

σ1 = σci + σ3 tan2 (45 + ϕ/2) = 136.82 + 8*tan2 (45 + 89.31/2) = 202.29 MPa

σ3 = 15 MPa

σ1 = σci + σ3 tan2 (45 + ϕ/2) = 136.82 + 15*tan2 (45 + 89.31/2) = 259.82 MPa

d)

σ1 = 260 MPa

σ1 = σ3 + 128.06 {(21.71* σ3/128.06) + 1}0.5

260 = σ3 + 128.06 {(21.71* σ3/128.06) + 1}

σ3 = 5.81 MPa

e)

σ3 = 5.6 MPa

σ1 = σci + σ3 tan2 (45 + ϕ/2) = 136.82 + 5.6*tan2 (45 + 89.3/2)

= 182.74 MPa

The rock mass will be unstable as the existing axial stress is lower than the axial stress expected to cause failure.

f)

σ’ = σ – u

u = σ – σ’ = 5.6 – 5.8

= 0.8 MPa

Question Five

a)

Figure 4. Peak and residual envelopes.

b)

Cohesion and internal angle of friction

ɵp = 350 and ɵr = 430

c = 16 MPa

c)

ɵt = (2cCos ɵ)/(1+Sin ɵ)

= (2*16*Cos 35)/(1 + Sin 35)

= 16.66 MPa

ɵc = (2cCos ɵ)/(1- Sin ɵ)

= (2*16*Cos 35)/(1 - Sin 35)

= 61.47 MPa

ɵt’ = (cCos ɵ)/(2 – Sin ɵ)

= (16*Cos 35)/(2 - Sin 35)

= 9.19 MPa

d)

Figure 5. Deviator stresses squared versus σ.

(σ1’ – σ3’)2 = mi σci σ3’ + σci2

σci2 = 818.26

σci = 28.61 MPa

Thus,

mi = -7.3351/28.61

= 0.26 MPa

σti = σci/mi = 28.61/0.26

= 110.04 MPa

Figure 6. Peak H-B.