SECOND HOMEWORK ASSIGNMENT

4.8: Given the following partial product tree, explode, offset, and determine the gross and net requirements for components H, I, J, and K. There are other components, but they are not connected to this problem. The quantities required are shown in parentheses. The master production schedule calls for the completion of 60 Hs in week 3 and 80 in week 5. There is a scheduled receipt of 120 Is in week 2. There are 400 Js and 400 Ks available. All lot sizes are lot-for-lot.

|  | Week | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Part H Lead Time: 1 week | Gross Requirements Scheduled Receipts Projected Available Net Requirements Planned Order Receipt Planned Order Release |  | 60 | 60 0 60 60 | 0 80 | 80 0 80 80 |
| Part I Lead Time: 2 weeks | Gross Requirements Scheduled Receipts Projected Available Net Requirements Planned Order Receipt Planned Order Release |  | 120 120 0 160 | 0 | 160 0 160 160 |  |
| Part J Lead Time: 1 week | Gross Requirements Scheduled Receipts Projected Available 400 Net Requirements Planned Order Receipt Planned Order Release | 400 | 320 80 | 80 | 80 | 80 |
| Part K Lead Time: 1 week | Gross Requirements Scheduled Receipts Projected Available 400 Net Requirements Planned Order Receipt Planned Order Release | 400 80 | 480 0 80 80 |  |  |  |

4:18: It is Monday morning, and you have just arrived at work. Complete the following MRP record as it would appear Monday morning. Lead time is 2 weeks, and the lot size is 100.

| Initial MRP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Week | 1 | 2 | 3 | 4 | 5 |
| Gross Requirements Scheduled Receipts Projected Available 50 Net Requirements Planned Order Receipt Planned Order Release | 70 100 80 100 | 40 40 | 80 100 60 100 100 | 50 10 | 40 100 70 100 |

During the week, the following events occur. Enter them in the MRP record.

- The planned order for 100 in week 1 is released.

- Thirty of the scheduled receipts for week 1 are scrapped.

- An order for 30 is received for delivery in week 3.

- An order for 50 is received for delivery in week 6.

- The gross requirements of 70 in week 1 are issued

| Week | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Gross Requirements Scheduled Receipts Projected Available Net Requirements Planned Order Receipt Planned Order Release | 70 70 50 100 | 40 10 100 | 110 100 10 100 | 50 100 60 100 100 | 40 20 | 50 100 60 100 |

5.15: Using the following route information, open order information, and MRP planned orders, calculate the load on the work center

Routing: Part 123: Setup time=2 standard hours, Run time per piece= 3 standard hours per piece

Part 456: Setup time = 3 standard hours, Run time per piece = 1 standard hour per piece

| Open Orders for parts |  |  |  |
| --- | --- | --- | --- |
| Week | 1 | 2 | 3 |
| 123 | 12 | 8 | 5 |
| 456 | 15 | 5 | 5 |
| Planned Orders for parts |  |  |  |
| Week | 1 | 2 | 3 |
| 123 | 0 | 5 | 10 |
| 456 | 0 | 10 | 15 |

| Load Report |  |  |  |
| --- | --- | --- | --- |
| Week | 1 | 2 | 3 |
| Released load 123 | 2+(12*3)=38 | 2+(8*3)=26 | 2+(5*3)=17 |
| 456 | 3+(1*15)=18 | 3+(1*5)=8 | 3+(1*5)=8 |
| Planned load 123 | 2+(0*3)=2 | 2+(5*3)=17 | 2+(10*3)=32 |
| 456 | 3+(0*1)=3 | 3+(10*1)=13 | 3+(15*1)=18 |
| Total load | 61 | 64 | 75 |

5.16: Complete the following load report and suggest possible courses of action

| Week | 18 | 19 | 20 | 21 | Total |
| --- | --- | --- | --- | --- | --- |
| Released load | 160 | 155 | 100 | 70 | 485 |
| Planned load | 0 | 0 | 70 | 80 | 150 |
| Total load | 160 | 155 | 170 | 150 | 635 |
| Rated Capacity | 150 | 150 | 150 | 150 | 600 |
| (Over)/Under Capacity | -10 | -5 | -20 | 0 | -35 |

The work center is overloaded. There is need to adjust the load by changing the priority plan. The available capacity should be increased to meet the high total load.

5.18: Back schedule the following shop order. All times are given in days. Move time between operations is 1 day, and wait time is 1 day. Due date is day 200. Assume orders start at the beginning of a day and finish at the end of a day.

| Operation number | Work Center | Operation time (days) | Queue time (days) | Arrival date (a.m.) | Finish date (p.m.) |
| --- | --- | --- | --- | --- | --- |
| 10 | 110 | 3 | 2 | 173 | 181 |
| 20 | 120 | 5 | 3 | 183 | 190 |
| 30 | 130 | 3 | 2 | 192 | 198 |
|  | Stores |  |  | 200 |  |

We schedule back from the due date 200. Operation 30 must be done on day 198 because of one-day wait time and one-day move time. This operation requires 3+2+1= 6 days, and thus it must start on 198-6= 192. Operation 20 requires seven days (2+4+1) and thus must begin on 190-7=183. Operation 10 takes eight days (4+3+1), and thus must start on 181-8=173.