Access Lists

Is a rule that we can use to filter traffic across the network

It act as a firewall that can deny or allow some traffic to enter into the network

It helps us to deny or permit based on the source IP address

Steps to create access lists

Create the access lists. Can be standard or extended

Apply the access list to an interface on a router either inbound or outbound

Inbound headed to the router and outbound headed outside the router

Standard ACL is going to get the number (1-99) applied closest to the destination while extended ACL  gets (100-199) applied closest to the source

Standard ACL (1-99)

Denies or permit source IP address

Extended ACL (100-199)

Denies or permits source IP address

Denies or permits destination IP address

Denies or permits port(service)