Physics Lab 1 Report

Student’s Name
Department, University
Course Number and Name
Instructor’s Name
Date

Physics Lab 1 Report

Purpose

To determine the mass of brass and aluminum Cylinders and Cuboids

To determine the Volume of Cylinders and Cuboids

Objectives

- To measure the length height and width of cuboid

- To measure the diameter and height of cylinders

Equipment

- Beam Balance

- Caliper

- Ruler

- Micrometer

- Brass Cylinder

- Brass Cuboid

- Aluminum Cylinder

- Aluminum Cuboid

Theory and equations

We will do me measurements with the assumptions that there is no external factor affection the readings we get from the measurements. We will calculate the Percentage error in measurements using the Formula:

% error = [(Result-Accurate)/Accurate] * 100

We will determine the Density of all objects using the formula:

D=M/V

Where D=density, M=mass and V=Volume

We will determine the volume as follows:

Cylinders

V= πr2h Where V=volume, r=radius, h=height

Cubes

V=lwh Where V=volume, w=width, h=height

Procedures

We shall measure the lengths, heights and widths using the ruler in centimeters (cm)

We shall use the beam balance to measure the mass of the objects in grams (g)

We shall use the calipers to measure the smaller heights and Cylinder diameter in millimeters (mm) which will be converted to cm for calculations.

| Object | Height (cm) | Length (cm) | Width (cm) | Radius (cm) | Mass (g) |
| --- | --- | --- | --- | --- | --- |
| Brass Cuboid | 1.6 | 2.6 | 1.9 |  | 66.5 |
| Brass Cylinder | 6.36 |  |  | 22.23/10 = 2.223/2 = 1.1115 | 208.3 |
| Aluminum Cuboid | 1.6 | 4.9 | 3.2 |  | 66.2 |
| Aluminum Cylinder | 6.372 |  |  | 22.32/10=2.232/2 = 1.116 | 67.5 |

Data and Observations

Calculations

Brass Cuboid

Volume

= lwh

=2.6*1.9*1.6

= 7.904 cm3

Density

=m/v

=66.5/7.904

=8.413g/cm3

Aluminum Cuboid

Volume

=lwh

=4.9*3.2*1.6

=25.088 cm3

Density

=m/v

= 66.2/25.088

=2.639 g/cm3

Brass Cylinder

Volume

= πr2h

= 3.14159*1.11152 *6.36

= 24.685 cm3

Density

=m/v

=208.3/24.685

=8.438 g/cm3

Aluminum Cylinder

Volume

= πr2h

= 3.14159*1.1162*6.372

=24.932 cm3

Density

=m/v

=67.5/24.932

=2.707g/cm3

Results

| Object | Volume (cm3) | Density (g/cm3) |
| --- | --- | --- |
| Brass Cuboid | 7.904 | 8.413 |
| Aluminum Cuboid | 25.088 | 2.639 |
| Brass Cylinder | 24.685 | 8.438 |
| Aluminum Cylinder | 24.932 | 2.707 |

Conclusions

The results show that even at a smaller volume, the Brass Cuboid is denser than the Aluminum Cuboid. The brass Cylinder also has a slightly smaller volume than the Aluminum Cylinder but is relatively denser. This means brass has a higher density than Aluminum