| GENERAL NOTES |  |  |  |
| --- | --- | --- | --- |
| porch kitchen lounge USH bedroom 1 bedroom 2 | GENERAL All dimensions are in mm unless otherwise specified. Drawing are not to scale only figured dimensions to be used. The contractor must check and verify all dimensions before commencement of any work. CONSTRUCTION All slab at floor bed to be poured over 1000 gauge polythene sheet on 50mm thick marrams blinding on hardcore. All soils under slab around and under foundation to be poisoned for termite control. CIVIL All soil on cut embankments to be stabilized. The slope not to exceed the natural angle of repose STUCTURAL All black cotton soil to be removed from all buildings and paved surfaces. For all R.C work refer to structural Engineers details Depth of foundation to be determined on site to S.E approval All walls less than 150 mm thick to be reinforced with hoop iron at every alternate course. All adjacent R.C work and masonry walls to be tied with strip iron every alternate cost. MECHANICAL All plumbing and drainage to comply with relevant local authority. SVP to be provided at the head of the drainage system. Drains passing beneath buildings and driveways to be encased by 150mm concrete surrounding. The storm drain pipes to comply with BS 556 specifications. All underground foul and waste drain pipes shall be UPVC to comply with BS 5255. All inspection chambers covers and framing shall be cast iron. Minimum slope in the drains pipe to be 1 in 100. No chases will b allowed for pipes. All testing of pipes must be completed before plastering. All mechanical works must coordinated with electrical and any conflict must be clarified before work begins. PV denotes permanent ventilation. ELECTRICALS All conduits must be laid before plastering. |  |  |
| LEGEND |  |  |  |
| SECURITY LIGHTS RECESSED CEILING LIGHTS SOLAR PANELS USH UNDER SINK HEATER |  | Revision History AT C |  |
|  |  | Company Title ARCHTECON CONSTRUCTION |  |
|  |  | #Project Name PROPOSED RESIDENTIAL DEVELOPMENT FOR NAME OF THE GROUP |  |
|  |  | DRAWING TYPE PLANS ,ELEVATIONS , SECTIONS & ROOF PLAN |  |
|  |  | Modified by Date ARCHTECON CONSTRUCTION Checked by Date Verified Engineer………. |  |
|  |  | Drawing Scale 1:100 |  |