Tensile Testing Lab Report

[Name]

Saginaw Valley State University

Department of Mechanical Engineering

Principles of Engineering Materials Laboratory (ME250L)

Abstract

The experiment conducted was to obtain the true stress-strain values Aluminium 7075 by performing the finite element simulation analysis. From the experiment, it was observed that there were different levels of accuracy in the simulation that resulted from various ductile fracture criteria. However, other factors may affect the fracture ductility such as strain, stress, and damaged values were also analyzed.

Analysis of how cold working, heat treatment and work hardening affect the mechanical properties of an element. This includes the yield strength, ultimate strength, strain hardening, and strain-to-fracture.

The experiment also aimed to determine the more accurate method of collecting data in a tensile test experiment. The two methods under scrutiny include cross arm displacement and extensometer displacement measurements.

A comparison of the experimental data results was compared to the literature values taking into account reasons for any differences and similarities.

Keywords: tensile, aluminum, strength, strain, stress

TENSILE TESTING LAB REPORT

Introduction

Aluminium 7075 is mostly used in aerospace materials and construction due to its high strength to weight ratio. However, there still exist challenges of this alloy despite its physical and chemical characteristics. For a safe design of engineering structures and components using Aluminium 7075, its tensile properties have to be conducted. Axial and longitudinal forces are applied on the specimen until failure occurs due to deformation.

Different scholars had conducted a tensile test on Aluminium to determine the true values of its stress and strain and to determine the ductile fracture using finite element criterion obtained by simulation. Scholars observed six types of failures. Jain conducted the Aluminium ductile fracture criteria (Jain, Allin, and Lloyd 1999) The destructive standards were discussed by Weng and Sun where they used forecasting and finite element analysis (Weng and Sun 2000). Komori analyzed finite element nodes that separated the ductile failure criterion. This method was used to predict the shear of sabotage (Komori 2005). Failure analysis and determining the fracture criterion was performed by Yang et al. (2005). The experimental calculations were conducted by Hambli and Reszka (2002). They also simulated different values of ductile fracture criteria known as C value. They later determined the Cockfront and Latham criterion which they found out to be 0.455. Hoa et al. (2005) used a finite element fracture zone to predict the metal formation.

To determine the strain-stress relationship on a material, a finite element software is used to simulate realistic results. After performing a tensile stress test and obtaining the generated engineering strain diagram, the true strain and stress of the diagram can be obtained using these formulas:

Ϭ=S (1+ δ)

ℰ= ln (1+ δ)

Where

- Ϭ=true stress

- δ= engineering strain

- ℰ= true strain

- S=engineering stress

Objectives

- To determine the yield strength, ultimate tensile strength, and elongation for aluminum samples in two different heat treat conditions

- To compare the use of crosshead and extensometer displacement measurements

- To examine how cold working increases the strength of an aluminum alloy

PROCEDURE

One 7075 Aluminium specimen that was cold worked and two 7075 aluminium that were heat treated by annealing them in air for three hours at 415 °C. Literature data of the mechanical properties; tensile strength, yield strength and ductility of the specimens was derived from the textbox’s appendix. The data was used to generate functions of engineering stress against engineering strain.

Drawbacks

The tensile test machine is very expensive, with its price being not less than $10000. Furthermore the machine is susceptible to failure in case of overload.

Before the tensile test was done, the load cell rated maximum capacity was checked so as not to overload the system. The highest tensile strength that the machine could tolerate was calculated by dividing the cross-sectional area of the gage length of the test specimen.

A spreadsheet containing time, load, crossarm extension and extensometer displacement was created based on the tensile test data generated by the loading cell machines software. This data was used to generate experimental graphs of engineering stress and strain. Comparisons of the experimental data and literature data were made. Also differences between the extensometer displacement and crossarm displacement were studied.

The following formulas were used to generate additional data;

P   = load

Ao = initial cross-sectional area of the specimen

s   = engineering stress = P / Ao

Lo = gauge length measured on the specimen (4 inches)

∆ L = crossarm extension

e1 = engineering strain measured from the crossarm displacement = ∆ L / Lo

L20 = extensometer knife edge separation before tensile test (2 inches)

L2   = extensometer knife edge separation at any time during the tensile test

e2 = engineering strain measured from the extensometer = (L2 - L20) / L20

True stress and strain were also calculated.

A = instantaneous cross-sectional area of the specimen

σ = true stress = P/A = s (e2 + 1)

ε 1 = true strain measured by crossarm displacement = ln [(∆ L + L0)/L0] = ln (e1 + 1)

ε 2 = true strain measured by extensometer = ln (L2/L20) = ln (e2 + 1)

The test was done in four phases. The first specimen, cold worked 7075 aluminium, was loaded to failure and the results labeled as sample1. The second specimen annealed aluminium was loaded to failure.in the third test, annealed 7075 aluminium specimen was work harden by loading it above its yield point. This was referred to as sample 3. The specimen was then loaded again but to failure. This was sample4. The results from the four tests were used to generate graphs and values of young’s modulus yield strength, ultimate strength, stress and strain were obtained.

RESULTS AND DISCUSSIONS

Results from the cold worked 7075 aluminium which was loaded to failure are in the table below.

Table 1 sample 1 results

Table 2 sample 2 results

Table 3 sample 3 results

Table 4 sample 4 results

Figure 1 strain verse stress for sample 1 and sample2

The above is a plot of two curves:  σ as a function ε 2 for sample1 and sample2.Cold worked aluminium has lower yield strength compared to annealed samples. However annealed aluminium has superior strain hardening qualities. The fact that annealing involves heating the material under considerably high temperatures, a uniform lattice is formed as it cools thus minimizing the internal stress and cracks. Therefore it has higher yield strength and higher strain-to-fracture ratio due to minimized internal stresses. It also can withstand higher ultimate load.

Figure 2 true stress verses true strain

Sample 4 offers the highest yield strength in comparison to the other specimen. This is because majorly annealed aluminum 7075 specimen was work hardened (sample 3) before the final experiment. This increased its yield strength at the expense of its strain hardening strength. Sample 3 offers higher yield strength as compared to sample 2 as well as strain hardening.

Meanwhile sample3 exhibited higher ultimate strength and strain-to-failure in comparison to the other samples. This is due to the fact that it does not undergo any prior loading therefore there are minimal internal stress within the sample.

Figure 3 stress and trues stress verse strain graph

The true stress being a function of engineering stress, the two graphs do not cross at zero. The two stresses are proportional to each other.

.

Figure 4 strain verse engineering stress of sample 1

Young’s modulus (crossarm displacement);

E=stressstrain=slope=2376-399.40.01949-0.01001=208GPa

Young” modulus (extensometer displacement);

E=stressstrain=slope=2376-399.4-3.2347+3.9056=2.9GPa

Cross arm displacement, e1 gives a more accurate value than the extensometer. Extensometer takes a recording of the average displacement. This explains why its values are less compared to cross arm displacement method. In addition, calibration of the extensometer is rather difficult.

The literature values differ from the experimental data values. This is because experimental data is inclusive of the defects within the specimen while the literature data assumes the specimen has zero defects. These defects mainly occur during the manufacturing process.

CONCLUSION

Finite element software was used to conduct a test on four pieces of aluminimu 7075 and determine their stress-strain relationships. Graphs from the data generated by the machine were used to plot graphs which aided in analyzing the mechanical properties of the specimens. Careful recording and reading of measurement and good calibrated gauges can reduce errors in the experiment.

EXPERIMENTAL ERRORS

Errors in the results from the data obtained could be due to faulty loading machine, machine flexes, measurement recording errors and calibration errors.

REFERENCES

Hambli, R., Reszka, M., 2002. Fracture criteria identification using an inverse technique method and blanking experiment. International Journal of Mechanical Sciences 44, 1349-1361.

Hoa, V.C., Seo, D.W., Lim, J.K., 2005. Site of ductile fracture initiation in cold forging: a finite element model. Theoretical and Applied Fracture Mechanics 44, 58-69.

Jain, M., J. Allin, J., D. J. Lloyd, D.J., 1999. Fracture limit prediction using ductile fracture criteria for forming of an automotive aluminum sheet. International Journal of Mechanical Sciences 41, 1273-1288.

Komori, K., 2005. Ductile fracture criteria for simulating shear by node separation method. Theoretical and Applied Fracture Mechanics 43, 101–114.

Weng, T.L., Sun, C.T., 2000. A study of fracture criteria for ductile materials. Engineering Failure Analysis 7, 101-125.

Yang, X., Chen, C., Hu, Y., Wang, C., 2005. Damage analysis and fracture criteria for piezoelectric ceramics. International Journal of Non-Linear Mechanics 40, 1204 -1213.